TAO

REFERENCE MANUAL

(Rel. 2.0 May 1995)

Quadrics  and TAO are trademarks of Alenia Spazio S.p.A.

APE100 and ZZ are project names of the I.N.F.N. (Istituto Nazionale di Fisica Nucleare)

First Edition: September 1994

Second Edition: March 1995

APE100 is the parallel computer architecture designed by the I.N.F.N.

Quadrics is the family of parallel computers of the Alenia Spazio S.p.A.

Edited by  E. Panizzi and V. Ghisalberti

## CONTENTS:

µ

I.	CONVENTIONS	1

II.	STRUCTURE	1

PART  I:

1.	IDENTIFIERS	3

2.	NUMERICAL CONSTANTS	4

2.1.	INTEGER NUMERICAL CONSTANTS	4

2.2.	REAL NUMERICAL CONSTANTS	4

2.3.	COMPLEX NUMERICAL CONSTANTS	5

3.	COMMENT LINES : !!	6

4.	THE LINE CONTINUES MARKER: "..."	7

5.	DATA TYPE DECLARATION STATEMENTS	8

6.	ARRAYS	9

6.1.	REFERRING TO ARRAY ELEMENTS	10

7.	LOCAL TO GLOBAL CONDITIONS: THE ALL, ANY AND NONE OPERATORS	12

8.	ARITHMETIC OPERATORS	13

9.	RELATIONAL OPERATORS	15

10.	LOGICAL OPERATORS	17

11.	THE <MATH> LIBRARY	18

12.	THE  <STDIO> LIBRARY	20

13.	I/O LISTS	21

14.	NODE SPECIFICATION	24

15.	SLICE SPECIFICATION	25

16.	TOPOLOGY SYSTEM VARIABLES	27

17.	RANDOM NUMBERS GENERATION	28

17.1.	THE <RANDOM> LIBRARY	28

17.2.	THE <RAND16> LIBRARY	28

18.	REMOTE ACCESS MECHANISM	29

19.	ZZ STATEMENTS	30

20.	OPERATORS	31

20.1.	_EXPR, _TERM, _FACT, _FUN	32

20.2.	RRETURN	39

21.	OPTIMIZATION BLOCKS	40

PART  II:

ABS	43

ACOS	45

ACOSH	46

ALL	47

ANY	48

ASIN	49

ASINH	50

AS LONG AS	51

ATAN	52

ATAN_0	53

ATAN_1	54

ATAN_ISQRT_3	55

ATAN_POS	56

ATANH	57

ATAN2	58

BINARY	59

BOARD_ID	60

BOARDS_NUM	61

BROADCAST	62

CALL	65

CAST_TO_REAL	66

CEXP	67

CLOSE	68

COMPLEX	69

COMPLEX (REAL_ARG)	70

CONSTANT	71

COS	72

COS_1OCT	73

COSH	74

CRATE_ID	75

CRATES_NUM	76

DO	77

DOWN	79

ELSE	80

ELSEWHERE	81

END	82

ENDDO	83

ENDIF	84

ENDRECORD	85

ENDWHERE	86

ENDWHILE	87

EXP	88

EXTRACT	89

FINT	91

FRONT	92

GET REMAINING TIME IN	93

GET TIME IN	94

I_BOARDS_NUM	95

I_CRATE_ID	96

I_CRATES_NUM	97

I_DEF_ABS_X, I_DEF_ABS_Y,   I_DEF_ABS_Z	98

I_MACHINE_LX, I_MACHINE_LY, I_MACHINE_LZ	99

I_RAN	100

IF	101

INIT_RANDOM	103

INTEGER	104

IS MADE OF	105

ISQRT	107

LEFT	108

LISTS	109

LOG	110

LOG10	111

MACHINE_LX, MACHINE_LY, MACHINE_LZ	112

MATRIX	113

MAX	117

MIN	118

MOD	119

MODULUS OF A COMPLEX NUMBER	120

MULTIDATA	121

MULTINDEX	122

N_OF	125

NODE_ABS_ID	126

NODE_ABS_X, NODE_ABS_Y and NODE_ABS_Z	127

NODE_ID	128

NONE	129

OPEN FOR READING	130

OPEN FOR WRITING	131

POW	132

R_RAN	133

RANDOM	136

READ	139

REAL	142

REAR	143

RECORD	144

REGISTER	148

REPEAT	150

REPLACE	152

RIGHT, LEFT, REAR, FRONT, UP, DOWN	154

RRETURN	156

SIGN	158

SIN	160

SIN_1OCT	161

SINH	162

SIZEOF	163

SQRT	165

STACK	166

SUBROUTINE	167

TAN	170

TANH	171

TEMPORARY	172

TRUNC	174

TYPE	175

UP	176

WHERE	177

WHILE	179

WRITE	181

!: LOGICAL  NOT OPERATOR	184

&	185

&&: LOGICAL  AND OPERATOR	187

.RE and .IM: REAL AND IMAGINARY PART   EXTRACTION POSTFIX OPERATORS	188

/BEEP	189

/FOR	190

/FOREACH	193

/IF	194

/INCLUDE	196

/PRINT	199

_EXPR	200

_FACT	201

_FUN	202

_TERM	203

| | : LOGICAL  OR OPERATOR	204

~: COMPLEX CONJUGATION OPERATOR	205

## I.	CONVENTIONSINDVOCE "CONVENTIONS"§

In this manual the following conventions are used in displaying the syntax requirements of user inputs to the system and in displaying examples:

- OPTIONAL ITEMS - An item enclosed in square bracket   ([ ]) is optional. To avoid ambiguities, italic characters are used to display syntax required square brackets ([ ]).

- COMMENTS - In examples, comment lines  are preceded by a double exclamation mark.

- LISTS - When an item is followed by a {, item} you can enter a single item or a number of items of the same type separated by commas.

## II.	STRUCTURE

This manual consists of two sections:

- PART I introduces the basic features of the TAO language.

- PART II contains a systematic description of the TAO statements.

The TAO statements are listed alphabetically.

# PART  I

## 1.	IDENTIFIERSINDVOCE "IDENTIFIERS"§INDVOCE "IDENTIFIERS"§

Identifiers are the symbolic namesINDVOCE "symbolic names"§ by which variables, constants, subroutines and external files are identified. They are strings of alphanumeric characters and underscores.

Valid identifiers begin with an alphabetic character.

In order to avoid syntax ambiguities, it is advisable not to use the language reserved keywords as identifiers.

The $ character INDVOCE "$ character "§is used in Tao predefined internal identifiers; in order to avoid unpredictable behaviours of the compiler, it is better not to use identifiers  including this character.

The TAO language is case insensitive.

Examples:

a, b1, c, delta

Q, W, PSI

one_time_two

N_2

## 2.	NUMERICAL CONSTANTSINDVOCE "NUMERICAL CONSTANTS"§INDVOCE "NUMERICAL CONSTANTS"§INDVOCE "NUMERICAL CONSTANTS"§

### 2.1.	INTEGER NUMERICAL CONSTANTS

INDVOCE "INTEGER NUMERICAL CONSTANTS"§

INDVOCE "INTEGER NUMERICAL CONSTANTS"§Integer numerical constants have values in the [-231; 231-1] range. They are optionally preceded by a plus or minus sign. If the sign is omitted the number is positive by default.

Examples:

12

-340

+22

### 2.2.	REAL NUMERICAL CONSTANTS

INDVOCE "REAL NUMERICAL CONSTANTS"§

INDVOCE "REAL NUMERICAL CONSTANTS"§Real numerical constants are represented as floating point numbersINDVOCE "floating point numbers"§ in the [-3.4x1038; 3.4x1038] range, according to the IEEE 754 single precision standard. They are optionally preceded by a plus or minus sign. If the sign is omitted the number is positive by default.

They may also be written as a positive or negative floating point number followed by an exponent part, consisting of the e letter followed by a signed or unsigned integer, which is the exponent of the power of ten to be multiplied by the preceding floating point number.

Examples:

0.34

+3.

-12.45

34.e-5

-3.4e6

### 2.3.	COMPLEX NUMERICAL CONSTANTSINDVOCE "COMPLEX NUMERICAL CONSTANTS"§

INDVOCE "COMPLEX NUMERICAL CONSTANTS"§Complex numerical constantsINDVOCE "Complex numerical constants"§ are represented as a couple of floating point numbers written between round brackets and separated by a comma, respectively standing for the real and the imaginary part of the complex number. They are stored as two real numbers.

The complex.hzt library predefines the following frequently used complex constants:

complex_i = (0.0 , 1.0)

complex_1 = (1.0 , 0.0)

complex_0 = (0.0 , 0.0)

complex_minus_i = (0.0 , -1.0)

complex_minus_1 = (-1.0 , 0.0)

Examples:

(1.0, 3.0)

(0.0, -3.0)

(-2.0, 0.0)

## 3.	COMMENT LINESINDVOCE "COMMENT LINES"§ : !!INDVOCE "!!"§

Description:

Two exclamation marks are used to insert comment lines in the source code. They are interpreted by the lexical analyser as an end of line; all the characters following them are ignored till the physical end of line is reached.

Examples:

!!       This is a comment line

write a         !! I am writing a

## 4.	THE LINE CONTINUES MARKER: "..."INDVOCE "\"...\""§

Description:

Three contiguous dots "..." are interpreted as a "line continues" marker, meaning that the line which they are written in has to be completed by the following one.

Examples:

real  a, b, c, ...

d, e, f, g, h, i, l

## 5.	DATA TYPE DECLARATION STATEMENTSINDVOCE "DATA TYPE DECLARATION STATEMENTS"§

Syntax:

data-type      data_name {, data_name}

Parameters:

data_type: integer, real, complex, or a user defined type.

data_name: the identifier of the declared variable.

Description:

Data type declaration statements define the data items to be used by your program, assigning their characteristics and associating them with symbolic names.

You can define any number of variables in one type declaration statement by separating their names with commas.

See also INTEGER, REAL, COMPLEX, RECORD, MATRIX.

Examples:

integer i, j, k

real alfa, beta

complex z, z1, z2

## 6.	ARRAYSINDVOCE "ARRAYS"§

Syntax:

unidimensional arrays:

data-type  array_name[N] {, array_name[N]}

multidimensional arrays:

data-type  array_name[N1,N2,...] {, array_name[N1,N2,...]}

Parameters:

data_type: integer, real, complex or a type previously defined by the user.

array_name: the array identifier.

N, N1, N2,...: integer numbers that specify the size of the array, that is the number of array items in each dimension.

Description:

An array consists of an ordered sequence of data items, all of the same type. It is identified with one name.

An array contains as many elements as specified in square bracket. Each element in the array is associated with an integer value that designates the position of the element in the sequence.

An array may extend in more than one dimension; to define multidimensional arrays, you have to specify the number of items in each dimension.

An array is always stored in contiguous memory locations. Multidimensional arrays are stored in the order of subscript progression, with the rightmost subscripts varying most rapidly (i.e. in twodimensional arrays successive elements of a row are stored in contiguous memory locations).

See also REFERRING TO ARRAY ELEMENTS and MULTINDEX.

### 6.1.	REFERRING TO ARRAY ELEMENTSINDVOCE "ARRAY ELEMENTS"§INDVOCE "REFERRING TO ARRAY ELEMENTS"§INDVOCE "REFERRING TO ARRAYS ELEMENTS"§

INDVOCE "REFERRING TO ARRAYS ELEMENTS"§

Syntax:

array_name[n1, n2,...] = element_value

Parameters:

array_name: the array identifier.

element_value: the value of the array element you are referring to.

ni: an integer number, having value in the [0, Ni-1] range (i = 1,2, ...). The 0 value identifies the first element of the array in the associated dimension, while the N-1 value refers to the last one.

Description:

An element in an array can be referenced by appending a subscript to the name of the array. Subscripts consist of integer numbers enclosed between square brackets, which designate the position of the element in the array. The subscript values must always be in the ranges defined in the array declaration statement and there must be as many subscripts as dimensions. An unpredictable behaviour may occur if the subscript values are out of their ranges; the user is not warned about it, unless the program was compiled with the "check boundary" option (+b).

They may also be expressed as integer variables, constants, or arithmetic expressions generating integer numbers.

See also ARRAYS and MULTINDEX.

Examples:

integer    i_vect[10], vect_4d[3,4,5,6]

real    r_vect[4], vect_3d[10,10,10]

complex    c_vect[12], vect_2d[4,4]

integer k

real a

complex z

constant index = 2

i_vect[0] = 4

i_vect[3] = k

vect_4d[1,1,1,1] = 9

r_vect[0] = a

r_vect[index] = 2.8

c_vect[8] = (1.0 , 2.0)

c_vect[8].re = 1.0

vect_2d[2,2] = z

## 7.	LOCAL TO GLOBAL CONDITIONS: THE ALLINDVOCE "ALL"§, ANY INDVOCE "ANY "§AND NONE INDVOCE "NONE "§OPERATORSINDVOCE "LOCAL TO GLOBAL FLOW\\: THE ALL, ANY AND NONE OPERATORS"§INDVOCE "LOCAL FLOW"§

The TAO language provides three operators that transform a set of local conditions (built up with local floating point variables) into a global condition.

They are called ALL, ANY and NONE; they have a local condition as operand and they give a global condition as result. This global result is obtained considering the local results collectively.

For example a global condition is evaluated as true when ALL the local conditions are true.

When one of these operators is used in the if, while or repeat constructs, the program flow is modified depending on the result of floating point local computations.

## 8.	ARITHMETIC OPERATORS

Syntax:

arith_elem     arith_operator      arith_elem

Parameters:

arith_elem: numbers, constants, or variables.

arith_operator: one of the following operators:

+INDVOCE "+"§         additionINDVOCE "addition"§

-INDVOCE "-"§          subtractionINDVOCE "subtraction"§

* INDVOCE "* "§         multiplicationINDVOCE "multiplication"§

/ INDVOCE "/ "§         divisionINDVOCE "division"§

Description:

Tao provides the basic arithmetic binary operators acting on integer, real and complex data; operations and expressions mixing integer and floating point data are not allowed, as these data are stored on different Quadrics memories.

For user defined record and matrix data types only addition and subtraction are automatically defined.

The operation to be performed is specified placing the operator between two arithmetic elements.

Any number of operations can be specified in the same line.

Operations are performed in the normal order of arithmetic evaluation: multiplication and division first, followed by addition and subtraction; the order of evaluation can be changed using parentheses. Operations of equal priority are performed left to right.

The plus and minus sign can be used as unary operators, in order to indicate positive and negative numbers or expressions.

Examples:

integer i, j, k, n, step

real a, b, c, d

complex z, z1, z2, z3, z4

record point

point x, y, w

k = i + j*n

step = step + 1

d = (b - c)/(2.0 + b)

a = b + z1.re

z = (z1 + z2)/(z1 - z2)

z4 = z1 * z2

x = y + w

## 9.	RELATIONAL OPERATORSINDVOCE "RELATIONAL OPERATORS"§

Syntax:

num_expr     relat_operator      num_expr

Parameters:

num_exp: numbers, arithmetic expressions, constants, or variables.

relat_operator: one of the operators listed below.

Description:

The program flow control conditions on Quadrics are always based on relational expressions between integer or floating point values. However, relational conditions mixing integer and floating point data are not allowed.

Tao provides the following relational operators which can be used to compare numbers and expressions:

= =INDVOCE "= ="§      equal

! =INDVOCE "! ="§       not equal

<         INDVOCE "<         "§less than

>         INDVOCE ">         "§greater than

< = INDVOCE "< = "§     less than or equal

> = INDVOCE "> = "§     greater than or equal

The comparison to be performed is specified placing the operator between two expressions.

Relational and arithmetic operations can be mixed up in expressions. Relational operations rank below arithmetic operations in order of evaluation.

Examples:

a = = b

a+b ! = c-d

g  <  h+l

z1.re >  z2.re

q < = 3.0

t > =  4.0

## 10.	LOGICAL OPERATORS

The execution of a block of statements may depend on more than one condition.

Conditions can be combined using the following boolean operators:

- logical OR operator: | |

- logical AND operator: &&

- logical NOT operator: !

See also | | , &&, ! in Part II.

## 11.	THE <MATH> LIBRARY

"Math.hztINDVOCE "Math.hzt"§" is the TAO predefined mathematical library.

It has to be included using the /include <math> statement.

The <math> library provides the following functions:

- ABS

- ACOS

- ACOSH

- ASIN

- ASINH

- ATAN

- ATANH

- ATAN_0

- ATAN_1

- ATAN_ISQRT_3

- ATAN_POS

- ATAN2

- CEXP

- COS

- COS_1OCT

- COSH

- EXP

- FINT

- ISQRT

- LOG

- LOG10

- MOD

- POW

- SIGN

- SIN

- SIN_1OCT

- SINH

- SQRT

- TAN

- TANH

- TRUNC

Other frequently used mathematical functions are provided by the $TAOLIB/start.hztINDVOCE "stdio.hzt"§ library, which is included by defaultINDVOCE "Math.hzt"§. They are:

- MAX

- MIN

The <math> library predefines also the following real constants:

constant  pigreco       !! p

constant  pig2            !! p/2

constant  pig4            !! p/4

constant  pig6            !! p/6

constant  i_pig4         !! 1/(p/4)

constant  i_sqrt3        !! 1/µ §

## 12.	THE  <STDIO>INDVOCE "<STDIO>"§ LIBRARY

The $TAOLIB/stdio.hztINDVOCE "stdio.hzt"§ library is included by default.

It defines a set of input/output instructions, used to transfer data between the variables of an executing program and an external medium (i.e. the user terminal and the disk files on the UNIX host computer).

The default input/output device is the user terminal on the UNIX host machine.

It is also possible to read data from a UNIX file or to write data on it; the file involved in this operation has to be opened and connected to a channel number, that will be used in the subsequent I/O operations on it.

The nature of the operation to be performed (reading or writing) has to be specified within the OPEN statement.

The statements available within the stdio.hzt library are:

- OPEN FOR READING

- OPEN FOR WRITING

- CLOSE

- WRITE

- READ

- BROADCAST

## 13.	I/O LISTSINDVOCE "I/O LISTS"§

Syntax:

var_name

quoted_string

array_item: nword

Parameters:

var_name: the identifier of the variable to be read or written.

array_item: an array element specified by the user.

nwords: the number of integer or floating point words to be read or written. If node or slice specifications are added nwords refers to the number of floating point words to be read or written for each single node.

quoted string : a string of alphanumeric characters put between quotes.

Description:

It is possible to specify a list of items to write or read.

The I/O list may be composed by one or more items. It consists of the names of all the objects involved in the input/output operation, separated by commas.

An I/O list may consist of:

- integer variables or array items;

- real variables or array items;

- complex variables or array items;

- real and imaginary parts of complex variables;

- blocks of array elements;

- quoted strings.

The default input/output formats are the following:

- integer variables: decimal

- real and complex variables: floating point

- strings: null terminated strings (an ASCII string

terminated by a  binary 0 value).

It is possible to read or write a block of array items starting from an element selected by the user. In this case, it is necessary to specify the number of integer or floating point words to be read or written. In a complex array the number of floating point words is twice that of a real array with the same dimensions, as the real and the imaginary parts of the array elements should be counted.

It is possible to read or write a list containing single items and blocks of array items together.

Data are read or written according to the following rules:

- Each item in the list is treated indepentently.

- All the data relative to the same item are written

sequentially.

See also READ, WRITE, BINARY, MULTIDATA.

Examples:

integer   i, k[2]

real   a, r_vet[10], m[3,3]

complex   z, c_vet[8]

!! Reading and writing a single item:

read i

read a

read from 3 z

read from 3 m[0,0]

write k[0]

write z.re

write on 4 c_vet[3]

!! Reading and writing  blocks of array items:

read from 5 m[0,0] : 6

read from 5  binary r_vet[0] : 2, c_vet[0] : 2

write on 6 c_vet[0] : 4, c_vet[6] : 2

write on 6 binary r_vet[0] : 3, c_vet[0] : 4

!! Reading and writing a list of items:

read  i, k[0], a, z, m[1,2]

read from 3 i, a, z, k[0] : 2

write  k[1], a, z.im, c_vet[2]

write on 6 binary z.re, r_vet[0] : 3, c_vet[0] : 4

!! Writing quoted strings:

write "I am going to write a"

write " a = " , a

do i = 0, 9

write "element n.", i, "=", r_vet[i]

enddo

## 14.	NODE SPECIFICATIONINDVOCE "NODE SPECIFICATION"§

Syntax:

[n1x, n1y, n1z]

INDVOCE "[n1x, n1y, n1z]"§

Parameters:

n1x: an integer number specifying the spatial coordinate of the node in the x direction.

n1y: an integer number specifying the spatial coordinate of the node in the y direction.

n1z: an integer number specifying the spatial coordinate of the node in the z direction.

Description:

A processing node of the machine is specified by means of its spatial coordinates in the x, y and z direction.

Examples:

write [0,0,0] a, b, c

read  [0,1,0] v[0] : 10

broadcast   a = b [0,0,0] : 1

## 15.	SLICE SPECIFICATION

INDVOCE "SLICE SPECIFICATION"§Syntax:

[n1x, n1y, n1z] [n2x, n2y, n2z]

INDVOCE "[n1x, n1y, n1z] [n2x, n2y, n2z]"§

Parameters:

n1x, n1y, n1z: three integer numbers respectively specifying the spatial coordinates of the first node in the x, y and z direction.

n2x, n2y, n2z: three integer numbers respectively specifying the spatial coordinates of the second node in the x, y and z direction.

Description:

A a set of nodes (or  slice) is specified by a pair of nodes. The two nodes identifying a slice must be the one with the lowest coordinates within the slice and the one with the highest ones.

Examples:

read [0,0,0][1,1,1] multidata a

!! one cube (Q1)

write [0,0,0][7,3,3] b

!! the lowest face of the tube (Q16)

write [0,0,0] [31,1,1] a, b

!! The I/O items (a and b) are treated separately; the slice is

!! scanned doing three nested loops, the inner of which is

!! along the z axis and the outer one is along the x axis.

!! The output will be:

!! value of a in node [0,0,0]

!! value of a in node [0,0,1]

!! value of a in node [0,1,0]

!! value of a in node [0,1,1]

!! value of a in node [1,0,0]

!! value of a in node [1,0,1]

!! value of a in node [1,1,0]

!! value of a in node [1,1,1]

!! value of b in node [0,0,0]

!! value of b in node [0,0,1]

!! value of b in node [0,1,0]

!! value of b in node [0,1,1]

!! value of b in node [1,0,0]

!! value of b in node [1,0,1]

!! value of b in node [1,1,0]

!! value of b in node [1,1,1]

!! The order is the same as NODE_ABS_ID.

!! It is different from the physical numbering of nodes.

write [0,0,0][1,1,1] node_abs_id

!! The output will be:

!! 0,0   1.0   2.0   3.0   4.0   5.0   6.0   7.0

## 16.	TOPOLOGY SYSTEM VARIABLES

The Quadrics operating system provides a set of initialized variables, containing information about the machine topology.

Some of these variables are initialized to integer values, in order to be used for different purposes by the Z-CPU; others are initialized to floating point values, to be used by the processing nodes of the machine.

The  integer variables storage space is allocated on the Z-CPU data memory, while the storage space for a copy of the floating point variables is reserved in the data memory of each node of the machine.

The integer topology variable names have the "I_" prefix.

The topology variables are:

- I_CRATES_NUM;

- I_BOARDS_NUM;INDVOCE "I_BOARDS_NUM"§

- I_CRATE_ID;

- I_MACHINE_LXINDVOCE "I_MACHINE_LX"§, I_MACHINE_LYINDVOCE "I_MACHINE_LY"§, I_MACHINE_LZ;INDVOCE "I_MACHINE_LZ"§

- I_DEF_ABS_XINDVOCE "I_DEF_ABS_X"§, I_DEF_ABS_YINDVOCE "I_DEF_ABS_Y"§, I_DEF_ABS_Z;INDVOCE "I_DEF_ABS_Z"§

- CRATES_NUM;INDVOCE "CRATES_NUM"§

- BOARDS_NUM;

- MACHINE_LXINDVOCE "MACHINE_LX"§, MACHINE_LYINDVOCE "MACHINE_LY"§, MACHINE_LZ;

- CRATE_ID;

- BOARD_ID;

- NODE_IDINDVOCE "NODE_ID"§

- NODE_ABS_XINDVOCE "NODE_ABS_X"§, NODE_ABS_YINDVOCE "NODE_ABS_Y"§,  NODE_ABS_Z;

- NODE_ABS_ID.INDVOCE "NODE_ABS_Z "§

INDVOCE "MACHINE_LZ"§

## 17.	RANDOM NUMBERS GENERATION

### 17.1.	THE <RANDOM> LIBRARY

INDVOCE "<RANDOM>"§Random.hzt INDVOCE "Random.hzt "§is a predefined language extension library, providing a pseudo random number generator.

It has to be included by the /include <random> statement.

The <random> library provides a system service which generates  a set of random numbers and allows to distribute them in several ways on the different nodes of the Quadrics machine.

### 17.2.	THE <RAND16> LIBRARY

INDVOCE "<RANDOM>"§Rand16.hzt INDVOCE "Random.hzt "§is a predefined language extension library, providing a pseudo random number generator.

It has to be included by the /include <rand16> statement.

The <rand16> library provides a number of statements, based on a parallel algorythm, that can be used to generate a set of random numbers and allows the user to initialize them in several ways on the different nodes of the Quadrics machine.

## 18.	REMOTE ACCESS MECHANISM INDVOCE "REMOTE ACCESS MECHANISM "§

The Quadrics architecture allows neighbouring nodes to exchange data. Each node is able to send data to its six nearest neighbours and to receive data from them. The external nodes of the machine array are connected between each other to preserve this kind of communication.

The memories of the six neighbouring nodes are visible on the addressing space of each node at addresses higher than the local ones.

Quadrics provides six predefined constant values to be summed to addresses in the local memory of a node, in order to refer to the higher addresses relative to the data memories of the neighbouring nodes. These constants are RIGHT, LEFT, REAR, FRONT, UP, DOWN.

## 19.	ZZ STATEMENTS

TAO is build up using the Zz dynamic parser; in fact Zz itself parses the TAO program to be compiled using the TAO grammar rules.

All Zz statements are thus available to the TAO programmer. He can use them to improve efficiency and expressive power in his programs.

The most useful Zz statements for the TAO programmer are explained in this manual - /FOR, /FOREACH, /IF, LISTS, /PRINT, /BEEP - as well as the way to build new operators with different precedences for user defined data types - _expr, _term, _fact, _fun.

The user should refer to the OPERATORS and OPTIMIZATION BLOCKS chapters in the first part of this manual and, for further study, to the "Zz , the TAO Engine" manual.

## 20.	OPERATORSINDVOCE "OPERATORS"§

The TAO language enables the programmer to introduce new operators and, if necessary, to overload the definition of predefined mathematical operators (to assign them a different meaning). It is possible to build operators acting on both variables of pre-defined data-types and variables of user defined data-types.

Operators belong to one of the following classes: prefix operators, postfix operatorsINDVOCE "postfix operators"§, n-infix operatorsINDVOCE "infix operators"§INDVOCE "n-infix operators"§, n-circumfix operatorsINDVOCE "circumfix operators"§INDVOCE "n-circumfix operators"§.

The following table shows these classes of operators and gives some examples:

| Class | Description | Examples |
| --- | --- | --- |
| prefix operatorsINDVOCE "prefix operators"§ | OP var | -a Tr a |
| postfix operators | var OP | z~ z.im z.re |
| n-infix operators | var OP var | a + b a * b |
| n-circumfix operators | OP var OP | (a, b) ¦ z ¦ f(a, b, c) |

Since many operators can be defined in more than one way, the user has a certain freedom in the choice of new operator class.

From now on, only operators acting on numerical variables are considered.

An operator can be thererfore defined as a symbol associated to the action that a scalar or vectorial function performes on scalar or vectorial variables.

Therefore, operators may be represented as functions acting on one or more variables of the same type or of different types:

type_dest = f(type_1, type_2, ...).

The result of the operator will be a value of the type_dest data- type.

Homogeneous operators, returning variables of the same type as input variables (i.e. type_dest =  type_1 =  type_2 = ...), can be distinguished from operators which mix different data-types.

### 20.1.	_EXPRINDVOCE "_EXPR"§INDVOCE "_EXPR"§, _TERMINDVOCE "_TERM"§INDVOCE " _TERM"§, _FACTINDVOCE "_FACT"§INDVOCE "_FACT"§, _FUNINDVOCE "_FUN"§

INDVOCE "_FUN"§

The TAO language allows the user to declare new data-types, starting from elementary data types, and to build compound data structures, formed by different type variables (See RECORD and MATRIX).

After declaring a new data type, it is possible to define new operators acting on variables of the new type; moreover, it is possible to overload the default definitions of mathematical operators.

When one introduces a set of new operators, he may need to assign them a certain order of evaluation. In other words, the user may wish to associate a level of precedence to every operator, so that some operations would be performed before others in the expressions which contain more than one operator.

When a data type is declared, TAO introduces a set of synctactical links to be used in the definition of new operators acting on variables of the new type, in order to preserve the desired evaluation precedence.

These synctactical links are listed below in their order of evaluation:

·	_fun

·	_fact

·	_term

·	_expr

As an example the method used to achieve the order of evaluation of arithmetic operators is shown.

Arithmetic operators acting on real variables are predefined by TAO; as far as they are concerned, the operations are performed in the normal order of arithmetic evaluation: multiplication and division first, followed by addition and subtraction; in other words, an expression like a + b * c is evaluated in the following order: first b is multiplied by c, then the result is added to a.

This order of evaluation is achieved by defining the "+" and "*" operators in the following way:

/real_expr -> real_expr^a "+" real_term^b {

temporary real result

...

... 	!! the lines of code which produce the

...	!! result

...

rreturn result

}

/real_term -> real_term^a "*" real_fact^b {

temporary real result

...

... 	!! the lines of code which produce the

...	!! result

...

rreturn result

}

Therefore, real_expr ranks below real_term in order of evaluation.

On the other hand, real_fun is associated to a higher level of precedence. The following example shows how to define a circumfix operator which will be evaluated before performing the arithmetic operations.

(From now on, for a matter of simplicity, the synopsis of the semantic action described by means of the TAO code inside curly brackets { } will be omitted.)

/real_fun -> "sin(" real_expr^a ")"

Within an expression, the sine of a real variable will be evaluated before performing the arithmetic operations.

The following syntactic scheme shows how homogeneous operators could be defined in order to achieve different levels of precedence.

type stands either for a predefined data type or for a type declared by the user before introducing the new operators.

OP is the symbol associated to the operator.

- PREFIX OPERATORSINDVOCE "PREFIX OPERATORS"§:

The standard way to define a prefix operator is:

/type_fact-> OP type_fact^a

The level of precedence achieved is the same as the unary "-" operator.

On the contrary, type_fun is associated to an higher level of precedence:

/type_fun -> OP type_fun^a

-  POSTFIX OPERATORSINDVOCE "POSTFIX OPERATORS"§:

The standard way to define a postfix operator is:

/type_fun -> type_fun^a OP

An operator defined in this way has the same level of precedence as the complex conjugation postfix operator "~"

It is possible to introduce a postfix operator having a lower level of precedence by means of :

/type_fact -> type_fact^a OP

- 2-INFIX OPERATORSINDVOCE "2-INFIX OPERATORS"§:

A 2-infix operator having the same level of precedence of the "+" operator can be defined by means of

/type_expr -> type_expr^a OP type_term^a

It will rank below

/type_term -> type_term^a OP type_fact^a

in order of evaluation, as this operator has the same level of precedence as the "*" operator.

- N-INFIX OPERATORSINDVOCE "N-INFIX OPERATORS"§:

N-infix operators can be introduced following the scheme used for 2-infix operators.

The following expressions respectively have the same order of evaluation as the "+" and "*" operators:

/type_expr->type_expr^a OP1 type_expr^b OP2 ...

OPN type_term^n

/type_term->type_term^a OP1 type_expr^b OP2...

OPN  type_fact^n

- N-CIRCUMFIX OPERATORSINDVOCE "N-CIRCUMFIX OPERATORS"§:

The standard way to introduce a n-circumfix operator is:

/type_fun ->OP1 type_expr^a OP2 type_expr^b...OPN

(e.g. /real_fun-> "sin("real_expr ")")

This class of operators ranks above "+" and "*" operator in order of evaluation.

- OPERATORS MIXING DIFFERENT TYPES:

As far as operators mixing different data-types are concerned, a safe approach is to adopt circumfix operators:

/type1_fun -> OP1  type2_expr^a  OP2

In this way a high level of precedence can be achieved.

Operators mixing different types can also be defined using hierarchic schemes similar to that described for homogeneous operators.

WARNING:

It is advisable not to use DO, WHILE, IF and REPEAT statements within the definition of new operators as they cause the lost of  register and temporary variables values.

See also OPTIMIZATION BLOCKS and REGISTER and TEMPORARY in Part II.

Examples:

!! Point type operators:

record point

real x, y, z

endrecord

!!  definition of the "+" operator for point type variables.

!!  The result is a point type variable whose fields are

!!  obtained summing the corresponding fields of the

!!  operands:

/point_expr -> point_expr^a "+" point_term^b {

temporary point ris

ris.x = a.x + b.x

ris.y = a.y + b.y

ris.z = a.z + b.z

rreturn ris

}

!!  definition of the "*" operator for point type variables.

!!  The result is a point type variable whose fields are

!!  obtained as products between the corresponding

!!  fields of the operands:

/point_term -> point_term^a "*" point_fact^b {

temporary point ris

ris.x = a.x * b.x

ris.y = a.y * b.y

ris.z = a.z * b.z

rreturn ris

}

!! Definition of the absolute value of a complex number:

/real_fun -> " | " complex_expr^_a " | " {

temporary  real  _ris

_ris = _a.re*_a.re + _a.im*_a.im

_ris = sqrt(_ris)

rreturn _ris

}

### 20.2.	RRETURN

The RRETURN statement is often used in the definition of Zz statements and operators.

An internal variable is created at the beginning of Zz action and a new value is assigned to it every time RRETURN is used; at the end of the action the final value of this variable is returned.

When RRETURN is used more than once inside a single statement, only the final value of the variable is returned at the end. The Zz action is always executed till the end; a RRETURN statement does not end the parsing of the action.

See also RRETURN in Part II.

## 21.	OPTIMIZATION BLOCKSINDVOCE "OPTIMIZATION BLOCKS"§

We call OPTIMIZATION BLOCK or SECTIONINDVOCE "SECTION"§ every group of consecutive instructions written between a couple of the following statements:

- DO;

- ENDDO;

- WHILE;

- ENDWHILE;

- IF;

- ENDIF;

- REPEAT;

- AS LONG AS;

- I/O instructions.

All these statements produce branches or entry points at the assembler level.

During the execution of a section, all the variables required are loaded from the node local memory into the MAD register file and their values are retained at most till the last statement in the optimization block is reached. At the end of the section, the values of the static variables are saved in the data memories, while the variables declared as register lose their content.

Therefore, every time a section starts or ends, the machine spends time computing the variables addresses and transfering data between the memory and the register file. We remark that during this time the MAD may be idle, as the ZCPU may not succeed in completing these operations while the MAD performes the floating point calculations.

For these reasons, it is important to achieve a high degree of efficiency inside each section, in order to obtain a short total run-time.

We list below some methods which can be used to improve the optimization of a tao code:

- Use of the /FOR statement.

Improvements in the optimization may be often achieved replacing DO-loops by /FOR loops.

- Use of REGISTER variables.

It is advisable to declare as register the variables that are often used within a program section.

- Use of the EXTRACT and REPLACE  tools for handling

blocks of data stored in contiguous memory locations. Faster I/O is performed in this way.

- Use of the MULTINDEX tool.

Multindex variables may be used in order to obtain a better degree of optimization in the computation of array indexes.

- Use of ZZ statements.

ZZ statements can be often used to obtain a more compact and optimized code. We remark that if the variables involved in the definition of new statements and operators are declared as temporary, they will be stored on the MAD registers, without being automatically saved in the data memories; in this way, unnecessary memory accesses are not performed during the execution.

# PART  II

## ABSINDVOCE "MOD"§

Syntax:

/include <math>

ABS (arg)

Parameters:

arg: an integer, real or complex expression.

Description:

The ABS function returns the absolute value of the argument.

Examples:

integer i, j, k

i = 2

j = - 4

k = abs(i)		!! k = 2

k = abs(j)		!! k = 4

real a, b, c

a = -2.0

b = 1.0

c = abs(a)		!! c = 2.0

c = abs(b)		!! c = 1.0

complex z

z = (1.0, 1.0)

d = abs(z)		!! d = µ §

!!  The absolute value of a complex number z = (z.re, z.im)

!!  is the real value (z.re*z.re + z.im*z.im)1/2.

## ACOSINDVOCE "COS"§

Syntax:

/include <math>

ACOS(arg)

Parameters:

arg: a real expression.

Description:

The ACOS function returns the arc cosineINDVOCE "cosine"§ of the argument. The result is expressed in radians.

Examples:

real a, b

b = acos(a)

## ACOSHINDVOCE "COS"§

Syntax:

/include <math>

ACOSH(arg)

Parameters:

arg: a real expression greater than -103.

Description:

The ACOSH function returns the hyperbolic arc cosineINDVOCE "cosine"§ of the argument. The result is expressed in radians. Due to the single precision rounding errors may occur if an argument smaller than -103 is used.

Examples:

real a, b

b = acosh(a)

## ALL

Syntax:

ALL (local_condition)

Parameters:

local_conditionINDVOCE "local condition"§: a condition defined on local floating point variables. Local conditions are defined using relational operators, comparing floating point numerical values or expressions. Logical operators can be used to combine conditions together.

Description:

The ALL operator returns a "true" global result if the local condition is evaluated as true on all the processing nodes. This means that it is possible to execute a block of instructions if, while, or as long as the specified local condition is true on all the nodes.

See also IF, WHILE and REPEAT.

Examples:

real a, b, c

if (all (a != 0.0))

b = c/a

else

b = 0.0

endif

## ANYINDVOCE "ANY"§

Syntax:

ANY (local_condition)

Parameters:

local_conditionINDVOCE "local condition"§: a condition defined on local floating point variables. Local conditions are defined using relational operators, comparing floating point numerical values or expressions. Logical operators can be used to combine conditions together.

Description:

The ANY operator returns a "true" global result if the local condition is evaluated as true on at least one node of the machine. It is then possible to execute a block of instructions if, while, or as long as the specified local condition is true on at least one of the processing the nodes.

See also IF, WHILE and REPEAT.

Examples:

real a, b

integer i

while (any (a>b))

a = a - 1.0

i =  i + 1

endwhile

## ASININDVOCE "COS"§

Syntax:

/include <math>

ASIN(arg)

Parameters:

arg: a real expression.

Description:

The ASIN function returns the arc sineINDVOCE "cosine"§ of the argument. The result is expressed in radians.

Examples:

real a, b

b = asin(a)

## ASINHINDVOCE "COS"§

Syntax:

/include <math>

ASINH(arg)

Parameters:

arg: a real expression greater than -103.

Description:

The ASINH function returns the hyperbolic arc sineINDVOCE "cosine"§ of the argument. The result is expressed in radians. Due to the single precision rounding errors may occur if an argument smaller than -103 is used.

Examples:

real a, b

b = asinh(a)

## AS LONG AS

INDVOCE "AS LONG AS"§Syntax:

REPEAT

block

AS LONG AS (global_condition)

Parameters:

global_condition: a condition defined on global integer variables.

A global condition can also be obtained using the ALL, NONE and ANY operators, which analyze the simultaneous status (on all the processing nodes) of a condition defined on local floating point variables.

Global conditions are defined by means of relational operators, comparing numerical values or expressions.

Conditions mixing integer and floating point data are not allowed.

block: a block of executable statements.

Description:

The AS LONG AS statement terminates the block of instructions contained in a repeat construct.

See also REPEAT, WHILE, ANY, ALL, NONE, RELATIONAL OPERATORS and LOCAL TO GLOBAL FLOW.

## ATAN

Syntax:

/include <math>

ATAN(arg)

Parameters:

arg: a real expression.

Description:

The ATAN function returns the arc tangent of the argument. The argument is expressed in radians.

Examples:

real a, b

b = atan(a)

## ATAN_0

Syntax:

/include <math>

ATAN_0(arg)

Parameters:

arg: a real expression.

Description:

The ATAN_0 function returns the arc tangent of the argument in the neighbourhood of 0. The argument is expressed in radians.

Examples:

real a, b

b = atan_0(a)

## ATAN_1

Syntax:

/include <math>

ATAN_1(arg)

Parameters:

arg: a real expression.

Description:

The ATAN_1 function returns the arc tangent of the argument in the neighbourhood of 1. The argument is expressed in radians.

Examples:

real a, b

b = atan_1(a)

## ATAN_ISQRT_3

Syntax:

/include <math>

ATAN_ISQRT_3(arg)

Parameters:

arg: a real expression.

Description:

The ATAN_ISQRT_3 function returns the arc tangent of the argument in the neighbourhood of 1/µ §. The argument is expressed in radians.

Examples:

real a, b

b = atan_isqrt_3(a)

## ATAN_POS

Syntax:

/include <math>

ATAN_POS(arg)

Parameters:

arg: a real expression.

Description:

The ATAN_POS function returns the arc tangent of a positive argument. The argument is expressed in radians.

Examples:

real a, b

b = atan_pos(a)

## ATANHINDVOCE "COS"§

Syntax:

/include <math>

ATANH(arg)

Parameters:

arg: a real expression.

Description:

The ATANH function returns the hyperbolic arc tangentINDVOCE "cosine"§ of the argument. The result is expressed in radians.

Examples:

real a, b

b = atanh(a)

## ATAN2

Syntax:

/include <math>

ATAN(arg1, arg2)

Parameters:

arg1: a real expression.

arg2: a real expression.

Description:

The ATAN2 function returns the arc tangent of the quotient of the two arguments (the first argument is divided by the second). The arguments are expressed in radians.

If the first argument is positive, the result is positive; if the first argument is negative, the result is negative.

If both arguments are zero, the result is zero. If the second argument is zero, the result is p/2 if the first argument is positive and -p/2 if the first argument is negative.

Examples:

real a, b, c

c = atan2(a, b)            !! c = atan(a/b)

## BINARYINDVOCE "BINARY"§

Syntax:

WRITE [ON file_unit] [nod_spec] [nod_spec] ... 	BINARY io_list

READ [FROM file_unit] [nod_spec] [nod_spec] ... 	[MULTIDATA] BINARY   io_list

Description:

The BINARY qualifier is added to perform an unformatted data input/output, that is to read or write data using the machine binary format. Data are transferred byte by byte, without translation, between the external unit and the elements of the i/o list (or viceversa), in their order of specification.

The binary and multidata qualifiers can be added in any order before the io_list. (i.e. both syntaxes MULTIDATA BINARY and BINARY MULTIDATA are allowed).

See also READ, WRITE and the STDIO library.

Examples:

read from 3 [0,0,0][0,1,1] binary a

write on 4 [0,0,0][1,1,1] binary v[0,0] : 4

## BOARD_ID

BOARD_ID is initialized to the same value on all the nodes of the machine belonging to the same board. This variable contains the identification number of the board to which the node belongs.

## BOARDS_NUM

BOARDS_NUM is initialized to the same value on all nodes. It contains the total number of processing boards forming the machine.

## BROADCASTINDVOCE "BROADCAST"§

Syntax:

BROADCAST dest_var = source_var  [node_spec]...

[node_spec] : nwords

Parameters:

dest_var: the destination variable.

source_var: the source variable.

node_spec: three integer  numbers in square brackets, separated by commas, identifying a processing node.

(See also NODE SPECIFICATION and SLICE SPECIFICATION).

nwords: the number of floating point words to be copied from the source node. If a source slice is specified, nwords refers to the number of floating point words to be copied from each single node in the specified slice.

Description:

The BROADCAST statement is used to copy the value of a floating point variable, stored in a particular node or slice, to all the nodes in the machine. The source and destination variables involved in this operation have to be specified, together with the number of floating point words to be copied from the source node. If a slice is specified nwords refers to the number of floating point words to be copied from each single node of the specified slice. It is possible to copy a block of array elements starting from an element specified by the user.

If node and slice specifications are omitted floating point data are copied from the default node in the default processing board.

Examples:

real a, b

broadcast   a = b [0, 0, 0] : 1

!! the value of b stored in the [0,0,0] node is copied to all

!! the processing nodes and stored in a.

real c, v[4]

broadcast   v[0] = c [0, 0, 0][1, 1, 0]: 1

!! the values of c stored in the 4 nodes in the [0,0,0][1,1,0]

!! slice are copied to the vector v. A copy of v is stored

!! in each node of the machine.

real v1[4], v2[16]

broadcast   v2[0] = v1[0] [0, 0, 0][1, 1, 0] : 4

!! the values of v1 stored in the 4 nodes in the [0,0,0][1,1,0]

!! slice are copied to the vector v2. v2 must be composed

!! of at least 16 elements A copy of v2 is stored in each

!! node of the machine.

real v1[4], v2[4]

broadcast   v2[0] = v1[0] [0, 0, 0] : 4

!! the values of the 4 elements of v1 stored in the [0,0,0]

!! node are copied to the vector v2. A copy of v2 is stored

!! in each node of the machine.

## CALLINDVOCE "CALL"§

Syntax:

CALL  sub_name  (arg_list )

Parameters:

sub_name: the subroutine identifier

arg_list: the list of the variables or values to be passed to the subroutine. Integer, real, complex, matrix and record type variables are allowed. The arguments must be separated by commas and must agree in order, number and type with the formal parameters declared in the subroutine statement; the names used may also be different.

Description:

The CALL statement transfers control and passes arguments to a subroutine. Parameters are passed to subroutines by address, by default; when a variable is passed by address, if its value is changed during the execution of the subroutine, this change will have effect also outside the subroutine. It is possible to pass a parameter by value preceding its name by the # character.

Examples:

call product(V, W, Q)

call mod(p1x, p1y, p2x, p2y, dist)

call mod(z1, z2,  dist1)

call delta( a , i , # k)                     !! k is passed by value

## CAST_TO_REALINDVOCE "CAST_TO_REAL"§

INDVOCE "CAST_TO_REAL "§Syntax :

CONSTANT  f_const = CAST_TO_REAL (i_const_name)

CONSTANT  f_const = CAST_TO_REAL (i_const_value)

Parameters:

i_const_name : the integer constant identifier.

i_const_value: an integer number.

f_const : the floating point constant identifier.

Description:

This TAO command converts a previously defined integer constant into a floating point one (to be stored in each node of the machine).

Examples:

constant    one = 1,  two = 2

constant    one_f = cast_to_real(one)

constant    two_f = cast_to_real(2)

constant    three_f = cast_to_real(one + two)

constant    three_f = cast_to_real(1 + 2)

!! one_f = 1.0

!! two_f = 2.0

!! three_f = 3.0

## CEXPINDVOCE "EXP"§

Syntax:

/include <math>

CEXP (arg)INDVOCE "exponential"§

Parameters:

arg: a complex expression.

Description:

The CEXP function returns e (arg) .

Examples:

complex z, w

w = exp(z)

!! w = ez

## CLOSE

Syntax:

CLOSE   file_unit

Parameters:

file_unit: an integer constant in the [2;15] range, specifying the channel number that has been assigned to the file..

Description:

This statement closes a file on the UNIX host machine.

Examples:

constant   file_in = 6

close   5

!! closes the file whose channel number is 5.

close   file_in

!! closes the file whose channel number is the value of

!! the constant file_in.

## COMPLEXINDVOCE "COMPLEX"§INDVOCE "COMPLEX"§

Syntax:

COMPLEX    compl_var_name {, compl_var_name}

Parameters:

compl_var_name: the identifier of the variable to be declared as complex.

Description:

This statement is used to declare a set of variables whose values are complex. Each of them is written as a couple of floating point numbers, representing its real and its imaginary part . The storage space for a copy of them is allocated on the data memory of each node in the machine.

Examples:

complex     z1, z2, c_vect[10]

!!  declaration of two complex variables named z1 and z2

!!  and of a vector of 10 complex numbers called c_vect.

## COMPLEX (REAL_ARG)INDVOCE "COMPLEX (REAL_ARG)"§INDVOCE "Z = COMPLEX (REAL_ARG)"§ INDVOCE "Z = COMPLEX (REAL_ARG) "§

Syntax:

COMPLEX (real_arg)

Parameters:

real_arg: the identifier of a real variable

Description:

The "complex" function converts its argument into a complex value. Its real part equals the argument value and its imaginary part is zero.

Examples:

real a

complex z

a = 1.0

z = complex(a)

!!    convertion of the real variable a into the complex

!!    variable z = (a, 0.0), that is z = (1.0, 0.0).

## CONSTANT

Syntax :

CONSTANT   name = value  {, name = value}

Parameters:

name : the constant value identifier.

value: an integer or a floating point number or expression.

Description:

The constant statement associates symbols to constant values INDVOCE "constant values "§or to expressions involving previously defined constants. Integer and floating point constants are allowed. Integer expressions in constant declarations are evaluated at compilation time; integer constants do not allocate memory space.

Examples:

constant    one = 1 , two = 2

constant    one_f = 1.0 , two_f = 2.0

constant    one_time_two = one*two

constant    one_f_time_two_f = one_f*two_f

!! To declare constant values you may also use the zz

!! syntax:

/one = 1

/two_f = 2.0

## COSINDVOCE "COS"§

Syntax:

/include <math>

COS(arg)

Parameters:

arg: a real expression.

Description:

The COS function returns the cosineINDVOCE "cosine"§ of the argument.

The argument is expressed in radians.

Examples:

real a, b

b = cos(a)

## COS_1OCTINDVOCE "COS_1OCT"§

Syntax:

/include <math>

COS_1OCT (arg)

Parameters:

arg: a real expression.

Description:

The COS_1OCT function returns the cosine of the argument in the [0, p/4] range. The argument is expressed in radians.

Examples:

real a, b

b = cos_1oct(a)

## COSHINDVOCE "COS"§

Syntax:

/include <math>

COSH(arg)

Parameters:

arg: a real expression.

Description:

The COSH function returns the hyperbolic cosineINDVOCE "cosine"§ of the argument.

Examples:

real a, b

b = cosh(a)

## CRATE_ID

CRATE_ID is initialized to the same value on all the nodes which belong to the same crate. This variable contains the identification number of the crate to which the node belongs.

## CRATES_NUMINDVOCE "CRATES_NUM"§

INDVOCE "CRATES_NUM"§CRATES_NUM is initialized to the same value on all nodes. It contains the number of crates (blocks of 128 nodes) the machine is composed of.

## DOINDVOCE "DO"§INDVOCE "DO LOOPS"§

Syntax:

DO   index_var =  start_val , stop_val  [, step_val]

block

ENDDO

Parameters:

index_var: the integer variable that controls the execution of the do loop. It is not necessary to declare it. It is possible to refer to the control variable inside the do loop either in expressions or in assignments. If it has not been declared, it becomes undefined after the enddo statement.

start_val: the initial value of the control variable. It is an integer value or an integer expression.

stop_val: the final value of the control variable. It is an integer value or expression.

step_val: the value by which the control variable is incremented after each loop. It is an integer value or expression. If omitted, it is set to 1 by default.

block: a block of executable statements.

Description:

The DO statement enables the user to execute a block of instructions iteratively until the value of a control variable equals or exceeds a terminal value. Then control transfers to the instruction immediately following the enddo statement.

It is not necessary to declare the control variable before using it.

It is possible to use the control variable within the do loop either in expressions or in assignments. When the control variable is not declared, it becomes undefined at the end of the do loop.

See also ENDDO and /FOR.

Examples:

do i = 1, 10

a = a + i

write "a =" , a

enddo

do  i = a+b, e, 2

q = i / (a+b)

write q

enddo

## DOWN

Description:

See RIGHT.

## ELSEINDVOCE "ELSE"§

Syntax:

IF (global_condition)

block 1

ELSE

block  2

ENDIF

Parameters:

global_condition: a condition defined on global integer variables.

A global condition can also be obtained using the ALL, NONE and ANY operators, which analyze the simultaneous status (on all the processing nodes) of a condition defined on local floating point variables.

Global conditions are defined by means of relational operators, comparing numerical values or expressions.

Conditions mixing integer and floating point data are not allowed.

block1, block2: blocks of executable statements.

Description:

The ELSE statement delimits a block of instructions within an if construct which will execute if the preceding block of statements has not been executed, that is if the global condition specified is not true. The block is terminated by an ENDIF statement.

See also IF and ENDIF.

## ELSEWHEREINDVOCE "ELSEWHERE"§

INDVOCE "ELSEWHERE"§Syntax:

WHERE (local_condition)

block 1

ELSEWHERE

block  2

ENDWHERE

Parameters:

local_conditionINDVOCE "local condition"§: a condition defined on local floating point variables. Local conditions are defined using relational operators, comparing floating point numerical values or expressions. Logical operators can be used to combine conditions together.

block, block1, block2: blocks of executable statements.

Description:

The ELSEWHERE statement is used, within where constructs, to perform a block of statements on those nodes where the preceding block of instructions has not been executed, that is where the local condition is not true.

The block is terminated by an ENDWHERE statement.

See also WHERE and ENDWHERE.

## ENDINDVOCE "END"§

Description:

The END statement terminates the block of instructions contained in a subroutine and transfers control to the main program instruction immediately following the call statement. See also SUBROUTINE, CALL.

## ENDDOINDVOCE "ENDDO"§INDVOCE "ENDDO"§

Syntax:

DO   index_var =  start_val , stop_val  [, step_val]

block

ENDDO

Description:

The ENDDO statement terminates the block of instructions contained in a do loop.

See also DO.

## ENDIFINDVOCE "ENDIF"§

Description:

The ENDIF statement terminates the block of instructions contained in an if construct.

See also IF and ELSE.

## ENDRECORDINDVOCE "ENDRECORD"§

Syntax:

RECORD    record_name

data_type  data_name {, data_name}

ENDRECORD

Parameters:

record_name: the record type identifier.

data_type : the elementary data-types forming the record. Real, complex, matrix and other record types are allowed.

data_name: the names of the fields composing the record.

Description:

ENDRECORD terminates the statements following the declaration of a record type.

See also RECORD.

## ENDWHEREINDVOCE "ENDWHERE"§

Description:

The ENDWHERE statement terminates the block of instructions contained in a where construct.

See also WHERE and ENDWHERE.

## ENDWHILEINDVOCE "ENDWHILE"§

Description:

The ENDWHILE statement terminates the block of instructions contained in a while construct.

See also WHILE.

## EXPINDVOCE "EXP"§

Syntax:

/include <math>

EXP (arg)INDVOCE "exponential"§

Parameters:

arg: a real expression.

Description:

The EXP function returns e (arg) .

Examples:

real a, b

b = exp(a)

!! b = ea

## EXTRACT INDVOCE "EXTRACT "§

Syntax:

EXTRACT   M_name   FROM   array_name[n]

Parameters:

M_name: the identifier of a matrix-type variable that has been declared as register. It is composed of N items.

array_name[n]: an array element.

Description:

The Tao language provides a set of useful statements to handle blocks of data stored in contiguous memory locations; they enable the user to manage the transfer of blocks of array items between the data memories and the MAD registers, in order to obtain more efficient codes.

The extract statement copies a block of N array items into a register variable of a previously defined matrix type, starting from the nth element of the array as specified by the user.

It is used to load groups of data that are stored in contiguous memory locations into the MAD registers. In this way unnecessary memory accesses are not performed during the execution.

See also REPLACE, MATRIX and REGISTER.

Examples:

matrix     complex    C.[5]

register    C     C1

complex      v_c[10]

extract    C1    from   v_c[0]

!! the first 5 elements of v_c are copied into C1

matrix      real    M_2d.[6,6]

register     M_2d     M2

real      v_2d[10,10]

extract    M2    from   v_2d[2,3]

!! a block of 36 elements is copied from v_2d into M2,

!! starting from v_2d[2, 3] to the v_2d[5, 8] included.

## FINTINDVOCE "FINT"§

Syntax:

/include <math>

FINT(arg)

Parameters:

arg: a real expression.

Description:

The FINT function returns a real number having the same sign as the argument; its value equals the largest integer whose absolute value does not exceed that of the argument. The FINT function returns the same result as the TRUNC function.

Examples:

real a, b, c, d, e, f, g, h

a = 3.2

b = 3.9

e = -3.3

g = -3.8

c = fint(a)	!! c = 3.0

d = fint(b)	!! d = 3.0

f = fint(e)		!! f = -3.0

h = fint(g)	!! h = -3.0

## FRONT

Description:

See RIGHT.

## GET REMAINING TIME IN

Syntax:

/include <rmq>

INTEGER	  int_var_name

GET  REMAINING  TIME  IN  int_var_name

Parameters:

int_var_name: the identifier of the integer variable into which the time is returned.

Description:

The GET REMAINING TIME function may be useful to RMQ (Quadrics Resource Manager) users. (See also "Quadrics Resource Manager" Manual).

This function returns how long the machine will be available to the current userINDVOCE "timer function"§.

The time is expressed in seconds and is returned in an integer variable provided by the user. If the time is unlimited -1 is returned.

Examples:

integer sec

get remaining time in sec

write "the time available is:", sec

## GET TIME IN

Syntax:

INTEGER	int_var_name

GET TIME IN    int_var_name

Parameters:

int_var_name: the identifier of the integer variable into which the time is returned.

Description:

This system service provides a  timer functionINDVOCE "timer function"§. It returns the time spent since the last call. The result is expressed in ms and is returned in an integer variable provided by the user. The first time you invoke this function it returns 0.

Examples:

integer delta

get time in delta

do i = 0, 9

a[i] = b[i] + c[i]

enddo

get time in delta

write "the time spent is:", delta

!! The time spent for the execution of the do-loop is

!!  returned in delta.

## I_BOARDS_NUM

Description:

INDVOCE "I_BOARDS_NUM"§

I_BOARDS_NUM contains the number of boards the  machine is composed of.

## I_CRATE_ID

Description:

INDVOCE "I_CRATE_ID"§This system variable is initialized  to a different integer value on the Z_CPU of each crate which compose a multicrate machine.

## I_CRATES_NUM

Description:

I_CRATES_NUM tells how many crates the machine is composed of.

A crate is a block of 128 nodes (16 processing boards) and a ZCPU controller.

## I_DEF_ABS_XINDVOCE "I_DEF_ABS_X"§, I_DEF_ABS_YINDVOCE "I_DEF_ABS_Y"§,   I_DEF_ABS_ZINDVOCE "I_DEF_ABS_Z"§

Description:

These system variables specify the x, y, and z coordinates of the default node. It is the node from which output operations are performed if neither a node nor a slice of the machine is specified within a write statement.

## I_MACHINE_LXINDVOCE "I_MACHINE_LX"§, I_MACHINE_LYINDVOCE "I_MACHINE_LY"§, I_MACHINE_LZ

Description:

INDVOCE "I_MACHINE_LZ"§

These system variables specify the machine dimensions in the x, y and z directions.

## I_RANINDVOCE "ISQRT"§

See R_RAN.

## IFINDVOCE "IF"§

Syntax:

1)

IF (global_condition)

block

ENDIF

2)

IF (global_condition)

block 1

ELSE

block  2

ENDIF

Parameters:

global_condition: a condition defined on global integer variables.

A global condition can also be obtained using the ALL, NONE and ANY operators, which analyze the simultaneous status (on all the processing nodes) of a condition defined on local floating point variables.

Global conditions are defined by means of relational operators, comparing numerical values or expressions.

Conditions mixing integer and floating point data are not allowed.

block, block1, block2: blocks of executable statements.

Description:

The IF statement executes an instruction or a block of instructions if the  global conditions are verified.

Two forms are possible as shown above :

1) The instructions contained in the block are executed if the global condition is true; otherwise control transfers to the instruction immediately following the endif statement.

2) The instructions of block 1 are executed if the global condition is true; otherwise the statements of block 2 are performed.

See also ANY, ALL, NONE, ELSE, ENDIF, RELATIONAL OPERATORS, LOCAL TO GLOBAL FLOW.

Examples:

if  (a > 5)

b = a + c

c = a

endif

if (g < 4)

b = a + g

else

b = a - g

endif

## INIT_RANDOM

Description:

See R_RAN.  INDVOCE "INTEGER  "§INDVOCE "INTEGER  "§

## INTEGER  INDVOCE "INTEGER  "§INDVOCE "INTEGER  "§

Syntax:

INTEGER    int_var_name {, int_var_name}

Parameters:

int_var_name: the identifier of the variable to be declared as integer.

Description:

This statement is used to declare one or more integer variables whose storage space is allocated on the Z-CPU data memory.

Examples:

integer    i, k, n[5], step

!!  declaration of three integer variables named i, k and

!!  step and of a vector of 5 integer numbers called n.

## IS MADE OFINDVOCE "ISQRT"§

Syntax:

MATRIX     data_type     matrix_name1.[M,N]

MATRIX     data_type     matrix_name2.[N]

TYPE  matrix_name1  IS MADE OF  matrix_name2

Parameters:

data_type:  real, complex or a previously defined record or matrix type.

matrix_name1, matrix_name2: the matrix types identifiers.

M, N: the number of  elementary fields forming the matrix types.

Description:

The IS MADE OF statement allows the user to reference a two-dimensional matrix type variable using only one index, obtaining an unidimensional matrix type variable.

See also MATRIX.

Examples:

matrix real square.[2,2]

matrix real line.[2]

type square is made of line

square  Q1

line  L1, L2, L3

L1.[0] = 0.0

L1.[1] = 1.0

L2.[0] = 2.0

L2.[1] = 3.0

!! The Q1 variable can be initialized in the following way:

Q1.[0] = L1

Q1.[1] = L2

!!  that is  Q1.[0,0] = 0.0,   Q1.[0,1] = 1.0,

!!  Q1.[1,0] = 2.0,  Q1.[1,1] = 3.0

!! At the same way L3 can be initialized writing:

L3 = Q1.[0]

!! that is  L3.[0] = 0.0 and L3.[1] = 1.0

## ISQRTINDVOCE "ISQRT"§

Syntax:

/include <math>

ISQRT(arg)

Parameters:

arg: a real expression.

Description:

The ISQRT function returns the reciprocal of the square root of the argument.

Examples:

real a, b

b = isqrt(a)

!! b = 1/µ §

## LEFT

Description:

See RIGHT

## LISTSINDVOCE "LISTS"§

Syntax:

/listINDVOCE "/list"§ _name = {item_list}

Parameters:

list_name: the list identifier. It is the name of the Zz variable which the list is assigned to.

item_list: the items belonging to the list. They can be variables of any type, numbers and quoted strings, separated by spaces.

Description:

TAO provides some facilities to manage lists of items at compilation time.

It is possible to identify a list of items with one name, by assigning it to a variable. Each item of the list is associated with an integer value that designates its position in the list; you can refer to any item in the list by specifying this number.

See also Zz STATEMENTS, /FOREACH.

Refer to "Zz, the TAO Engine" manual.

Examples:

/my_list = {1 2 3 4 5}

/print my_list.1                       !!  1 is printed on the screen

## LOGINDVOCE "LOG"§

Syntax:

/include <math>

LOG(arg)

Parameters:

arg: a real expression.

Description:

The LOG function returns the natural logarithmINDVOCE "logarithm"§ of the argument.

Examples:

real a, b

b = log(a)

!! b = ln(a)

## LOG10INDVOCE "LOG"§

Syntax:

/include <math>

LOG10(arg)

Parameters:

arg: a real expression.

Description:

The LOG10 function returns the common logarithmINDVOCE "logarithm"§ (base 10) of the argument.

Examples:

real a, b

b = log10(a)

!! b = log10(a)

## MACHINE_LXINDVOCE "MACHINE_LX"§, MACHINE_LYINDVOCE "MACHINE_LY"§, MACHINE_LZINDVOCE "MACHINE_LZ"§

Description:

These system variables are initialized to the same real value on all the processing nodes in the machine. They respectively specify the dimension in the x, y and z direction of the machine.

For a Q1:

MACHINE_LXINDVOCE "MACHINE_LX"§ = 2.0

MACHINE_LYINDVOCE "MACHINE_LY"§ = 2.0

MACHINE_LZ = 2.0

For a Q16:

MACHINE_LXINDVOCE "MACHINE_LX"§ = 32.0

MACHINE_LYINDVOCE "MACHINE_LY"§ = 2.0

MACHINE_LZ = 2.0

## MATRIXINDVOCE "MATRIX"§

Syntax:

Unidimensional matrix types:

MATRIX    data_type   matrix_name.[N]

Two-dimensional matrix types:

MATRIX    data_type   matrix_name.[M, N]

Declaration of matrix type variables:

matrix_name    name {, name}

Parameters:

data_type: real, complex or a previously defined record or matrix type.

matrix_name: the matrix type identifier.

N: the number of the elementary fields forming the matrix type.

M, N: the number of the elementary fields forming the matrix type in each dimension.

name: the names of the variables of the new type. Each of them is formed by as many fields as declared in the matrix statement.

Description:

This statement should not be confused with the declaration of conventional arrays (For this declaration statement see DECLARATION OF ARRAYS).

It is conceptually analogous to the RECORD statement, since it introduces the declaration of a new data type.

The MATRIX statement enables you to declare a new aggregate data type, starting from simpler floating point values. The new type is composed of as many floating point fields as specified by the number in square brackets. Unidimensional and two-dimensional matrixes are allowed. In the two-dimensional case the user must specify the number of elementary fields in each dimension.

Each element of a matrix type variable is associated with an integer number, having value in the [0; N-1] range; the 0 value identifies the first matrix element while the N-1 value refers to the last one.

You can access a particular element specifying its number within square brackets.

A matrix type may also consist of record and matrix types.

Declared a new matrix type it is possible to:

-  use it to build new aggregate types;

-  define register and temporary variables of the new type;

-  define multidimensional arrays of the new type;

- pass variables of the new type as arguments to a subroutine.

Matrix type variables cannot be passed to subroutines as values.

The use of an integer expression as an index identifying a particular matrix element (i.e. M.[i]) is allowed only for static matrix variables; you cannot use it for temporary and register matrixes .

See also RECORD.

Examples:

matrix    real    R_M.[10]

R_M     M1

!!  definition of  a new type called R_M, composed of

!!  10 real numbers. M1 is a variable of this type.

M1.[3] = 3.0

!!   The value of the fourth element of M is 3.0

matrix   complex  C_M.[2,2]

C_M    C1

!! C_M is a two-dimensional matrix type, composed of

!! 2x2 complex numbers. C1 is a variable of this type.

C1.[0,0] = (1.0,2.0)

C1.[0,0].re = 1.0

!! The [0,0] element of C1 is the complex number

!! (1.0,2.0). Its real part is 1.0.

record  point

real   x, y, z

endrecord

matrix   point  point_matr.[4]

point_matr   p_M

!!  definition of a new type named point_matr,

!!  composed of 4 point type variables.

!!  p_M is a variable of  point_matr type.

record big

real a, b

C_M C

endrecord

big alfa, beta

!!  definition of a new type named big, composed

!!  of 2 real fields and of a C_M type field;

!! alfa and beta are variables of  big type.

## MAXINDVOCE "MOD"§

Syntax:

MAX (arg1 , arg2)

Parameters:

arg1: an integer or a real expression.

arg2: an integer or a real expression.

Description:

The MAX function returns the largest of the two specified arguments.

Examples:

integer i, j, k

real a, b, c

i = 2

j = 4

a = -2.0

b = 1.0

k = max(i, j)		!! k = 4

c = max(a, b)		!! c = 1.0

## MININDVOCE "MOD"§

Syntax:

MIN (arg1 , arg2)

Parameters:

arg1: an integer or a real expression.

arg2: an integer or a real expression.

Description:

The MIN function returns the smallest of the two specified arguments.

Examples:

integer i, j, k

real a, b, c

i = 2

j = 4

a = -2.0

b = 1.0

k = min(i, j)		!! k = 2

c = min(a, b)		!! c = -2.0

## MODINDVOCE "MOD"§

Syntax:

/include <math>

MOD (dividend , divisor)

Parameters:

dividend: a real expression.

divisor: a real expression.

Description:

The MOD function divides the first argument by the second and returns the remainder.

Examples:

real a, b, c

c = mod(a, b)

## MODULUS OF A COMPLEX NUMBERINDVOCE "MODULUS OF A COMPLEX NUMBER"§INDVOCE "MODULUS OF A COMPLEX NUMBER"§

Syntax:

|compl_arg|

Parameters:

compl_arg: the identifier of a complex variable

Description:

The "|   |" function  returns the square of the absolute value of the argument, that is the real value z.re*z.re +z.im*z.im.

Examples:

real a

complex z

z = (1.0, 1.0)

a =  |z|

!! It returns a = z.re*z.re  + z.im*z.im = 2.0

## MULTIDATAINDVOCE "MULTIDATA"§

Syntax:

READ  [FROM file_unit]  nod_spec  nod_spec ...

MULTIDATA  [BINARY]  io_list

Description:

The MULTIDATA qualifier is used within READ statements to load different data on each processing node.

The first node considered in a read operation is the one having the lowest coordinates in the slice, while the last one is the node having the highest coordinates.

The binary and multidata qualifiers can be added in any order before the io_list. (i.e. both syntaxes MULTIDATA BINARY and BINARY MULTIDATA are allowed).

See also READ.

Examples:

real c[20]

read   from   6  [0, 0, 0] [1, 1, 1]  multidata c[0] : 20

!! 20 different real numbers are loaded in each node in the

!! specified slice (first board). c will have different values

!! on each node in the [0,0,0] [1,1,1] slice.

## MULTINDEXINDVOCE "MULTINDEX"§INDVOCE "MULTINDEX"§ INDVOCE "MULTINDEX "§

Syntax:

MULTINDEX  IN  [N1,N2,...,...]  i_name {, i_name}

i_name = [n1,n2,...,...]

array_name[i_name] = element_value

Parameters:

N1,N2,..: integer numbers specifying the size of the array.

i_name: the multindex variable identifier.

ni: an integer number having value in the [0, Ni-1] range  (i = 1,2, ...). The 0 value identifies the first element of the array in the associated dimension, while the N-1 value refers to the last one.

array_name: the array identifier.

element_value: the element value.

Description:

The MULTINDEX declaration statement manages the addressing inside multidimensional arrays in a fast and elegant way; it enables the user to select a particular array element without explicitely specifying its subscripts values, and to move around it without wasting a lot of machine time in the address computation of its neighbouring elements.

This statement defines one or more variables of a new type, used to access elements in a multidimensional array.

These variables are formed by as many integer numbers as the dimensions of the array they refer to; variables values have to be written between square brackets and separated by commas.

Multindex variables having the same dimensional structure can be summed to obtain a new multindex value.

Since Quadrics wastes much time in calculating the integer indexes of arrays, it is advisable to optimize the address computation in your code by the use of the multindex tool.

If you need to move in the neighbourhood of a fixed array element, you can do this by defining a suitable set of multindex variables. The addresses of the neighbouring elements may be calculated by adding appropriate multindex variables to the one identifying the starting element.

See also ARRAYS and REFERRING TO ARRAY ELEMENTS.

Examples:

real  v[3, 4, 5]

multindex in [3, 4, 5]  first

!! first is a multindex variable

first = [0,0,0]

v[first] =  0.0

!! The [0, 0, 0] element of v is v[first]: its value is 0.0

real M[20, 20], W[6]

multindex in [20, 20]  index, dx, dy, index_x, index_y

!! index, dx, dy, index_x and index_y are multindex

!! variables.

index = [0, 0]

dx = [1, 0]

dy = [0, 1]

M[index] = a

!!  M[index] = M[0,0] = a.

!!  You can move in the neighbourhood of [0, 0]

!!  incrementing its coordinates.

index = index + dx

M[index] = c*c

!!  M[index] = M[1, 0] = c*c

!!  You can do the same with dy

!!  An example of code in which it is necessary to

!!  move in the neighbourhood of a fixed array element

do i_x = 0, 5

do i_y = 0, 5

index = [i_x, i_y]

index_x = index + dx

index_y = index + dy

W[i_x] = M[index_x] + M[index_y]

enddo

enddo

## N_OFINDVOCE "N_OF"§

Syntax :

N_OF(array_name)

Parameters:

array_name: the identifier of an array.

Description:

The N_OF operator returns the number of the elements of the array passed as argument. It is evaluated at compilation time.

Examples:

integer     i_v[4]

real          r_v[3]

complex  c_v[2]

constant  c_i = n_of(i_v)                                  !! c_i = 4

constant  c_r = n_of(r_v)                                  !! c_r = 3

constant  c_c = n_of(c_v)                                 !! c_c = 2

## NODE_ABS_ID

Description:

NODE_ABS_IDINDVOCE "NODE_ABS_ID"§ is a system variable, inizialized to a different floating point value on each processing node of the machine. It represents the absolute identification number of the node.

For example, on Q1 machines, NODE_ABS_ID values range from 0 to 7, while on Q16 machines they range from 0 to 127.

On Q1 machines, however, NODE_ABS_ID values are different from NODE_ID values.

In fact the  NODE_ABS_ID value on each processing node depends on the lattice coordinates of that node and it is calculated in the following way:

node_abs_id = node_abs_z + node_abs_y*machine_lz +       		       +node_abs_x *machine_lz*machine_ly

## NODE_ABS_XINDVOCE "NODE_ABS_X"§, NODE_ABS_Y INDVOCE "NODE_ABS_Y"§and NODE_ABS_Z INDVOCE "NODE_ABS_Z "§

Description:

These system variables respectively specify the x, y and z spatial coordinate of a processing node.

## NODE_ID

Description:

INDVOCE "NODE_ID"§

NODE_ID is inizialized to a different floating point value (from 0 to 7) on each processing node within a board. This number specifies the physical number of a node within the board it belongs to.

## NONEINDVOCE "NONE"§

Syntax:

NONE (local_condition)

Parameters:

local_conditionINDVOCE "local condition"§: a condition defined on local floating point variables. Local conditions are defined using relational operators, comparing floating point numerical values or expressions. Logical operators can be used to combine conditions together.

Description:

The NONE operator returns a "true" global result if the local condition is evaluated as false on all the processing nodes (i.e. none of them is true). This means that it is possible to execute a block of instructions if, while, or as long as the specified local condition is false on all the processing nodes.

See also IF, WHILE and REPEAT.

Examples:

real a, b, c

repeat

a = sqrt(c*c - b*b)

b = b + 1.0

as long as (none (b*b > c*c))

## OPEN FOR READINGINDVOCE "OPEN FOR READING"§

Syntax:

OPEN FOR READING "file_name" AS file_unit

Parameters:

file_name: the name of the file to be opened.

file_unitINDVOCE "file_unit"§: an integer constant in the [2;15] range, specifying the channel number to assign to the file. The channels 0 and 1 are reserved by the operating system to serve the user terminal. These channel do not require open instructions.

Description:

The OPEN FOR READING statement opens an existing file on the UNIX host machine, allowing to read data from it. The file_unit is the channel number to refer to in the following I/O operations which involve the file.

Quadrics assumes as default directory for file input/output the current UNIX work directory of the user. A different directory has to be specified.

Examples:

open for reading "input.dat" as 5

open for reading "/usr3/new/read_file.dat" as 7

## OPEN FOR WRITINGINDVOCE "OPEN FOR WRITING"§INDVOCE "OPEN FOR WRITING"§

Syntax:

OPEN FOR WRITING "file_name" AS file_unit

Parameters:

file_name: the name of the file to be opened.

file_unitINDVOCE "file_unit"§: an integer constant in the [2;15] range, specifying the channel number to assign to the file. The channels 0 and 1 are reserved by the operating system to serve the user terminal. These channel do not require open instructions.

Description:

The OPEN FOR WRITING statement opens an existing file on the UNIX host machine, allowing to write on it. The file_unit is the channel number to refer to in the following I/O operations which involve the file. If the specified file does not exist, a new file having the same name is created in the current directory. If the file exists, it is rewritten.

Quadrics assumes as default directory for file input/output the current work directory. A different directory has to be specified by the user.

Examples:

open for writing "output.dat" as 6

## POWINDVOCE "MOD"§

Syntax:

/include <math>

POW (base, n)

Parameters:

base: a real expression.

n: a real expression.

Description:

The POW function raises the base to the nth power and returns the result.

Examples:

real a, b, c

a = 3.0

b = 2.0

c = pow(a, b)		!! c = 3.02 = 9.0

## R_RAN

Syntax:

Including the Rand16.hzt library:

/include <random>

/include <rand16>

Initialising the pseudo random number generatorINDVOCE "pseudo random number generator"§:

INTEGER   seed_var_name

seed_var_name = seed_var_value

INIT  RANDOM INDVOCE "SEED"§INDVOCE "RANDOM   SEED  IS"§ [qualifier] seed_var_name

Generating a floating point random number:

REAL  real_var

real_var = R_RAN

Generating an integer random number:

INTEGER  integer_var

integer_var = I_RAN

Parameters:

seed_var_name: the identifier of the seed variable. The seed variable has to be declared as integer and initialised before being used.

seed_var_value: the value to which the seed variable is initialised. For best results, it should be a large prime number.

qualifier: The BROADCAST qualifier is added to load the same floating point pseudo random number on all the processing nodes. The BOARD qualifier is added to load the same set of floating point pseudo random numbers on each processing board (but different on each node in the board).

integer_var: an integer variable.

real_var: a floating point variable.

Description:

The R_RAN statement is used to generate a floating point random number, while the I_RAN statement can be used to generate an integer random number.

Floating point random numbers have values between 0.0 and 1.0, while integer random numbers have values in the [0; 255] range.

If more than one random number is needed the R_RAN ( I_RAN) statement should be used more than once.

In the case of floating point random number generation, qualifiers are needed to specify whether the same set of numbers must be loaded on all nodes or on all the boards in the Quadrics machine.

If no qualifier is specified, a different floating point random number is loaded on each node of the machine.

Before being used, the generator has to be initialised, providing it with an integer "seed variable".

See also RANDOM.

Examples:

integer int_num, num,  num_board,  same_num,

integer j

real r, p, q

int_num = 87799005

init_random  int_num

j = i_ran

!! j is an integer random number

num = 21356799

init_random   num

r = r_ran

!! r has a different value on each node of the

!! Quadrics machine.

same_num = 156998345

init_random  broadcast same_num

p = r_ran

!! p has the same value on each node of the

!! Quadrics machine.

board_num = 692498345

init_random  board  board_num

q = r_ran

!! q has the same value on each board of the

!! Quadrics machine (but different on each node

!! within the same board).

## RANDOM

Syntax:

Including the Random.hzt library:

/include <random>

Initilizing the pseudo random number generatorINDVOCE "pseudo random number generator"§:

RANDOM   SEEDINDVOCE "SEED"§  ISINDVOCE "RANDOM   SEED  IS"§   seed_var_name

Generating integer random numbers:

RANDOM  integer_var [: n]

Generating floating point random numbers:

RANDOM INDVOCE "RANDOM "§ [qualifier] real_var [: n]

Parameters:

seed_var_name: the identifier of the seed variable. It has to be declared as integer and initialized to a value before being used. For best results, it should be initialized to a large odd value.

To generate different sets of random numbers, the seed variable should be initialized to a different value before each run.

qualifier: The BROADCAST qualifier is added to load the same set of floating point pseudo random numbers on all the processing nodes. The BOARD qualifier is added to load the same set of floating point pseudo random numbers on each processing board (but different on each node within the same board).

integer_var: an integer variable or an element of a previously defined integer array. If : n is specified the element is the starting element of the array which the random numbers are loaded into.

real_var: a real or complex variable or an  element of a previously defined real or complex array. If : n is specified the element is the starting element of the array which the random numbers are loaded into.

n: the number of random items to generate. It refers to the number of floating point or integer words; if omitted it is set to 1 by default.

Description:

The RANDOM statement produces a set of n integer or floating point pseudo random numbers with flat distribution. They are loaded in a predeclared integer, real or complex variable or array starting from the element specified in the statement.

It is not possible to generate a set of more than 32 random items. If more than 32 numbers are needed the pseudo random number generator should be invoked more than once.

Integer pseudo random numbers have values in the [0; 255] range, while floating point random numbers have values between 0.0 and 1.0.

In the case of floating point number generation, qualifiers are needed to specify whether the same set of numbers must be loaded on all nodes or on all the boards in the Quadrics machine.

If no qualifier is specified, a different set of floating point pseudo random numbers is loaded on each node of the machine.

Before being used, the generator has to be initialized, providing it with an integer "seed variable".

Examples:

integer number, i_vect[8]

real r_vect[32]

complex c_vect[16]

number = 21356799

random seed is number

random i_vect[0] : 8

random r_vect[0] : 32

random c_vect[0] : 32

## READINDVOCE "READ"§

Syntax:

Reading from the terminalINDVOCE "Reading from the terminal"§:

READ [nod_spec] [nod_spec] [MULTIDATA] ...

[BINARY]io_list

Reading from a fileINDVOCE "Reading from a file"§:

READ FROMINDVOCE "READ FROM"§ file_unit [nod_spec][nod_spec]...

[MULTIDATA] [BINARY] io_list

Parameters:

file_unit: an integer constant in the [2;15] range. It specifies the channel number assigned to the file.  (See also OPEN FOR READING)

node_spec: three integer numbers in square brackets, separated by commas, which identify a processing node. See also NODE SPECIFICATION and SLICE SPECIFICATION).

BINARY: the qualifier required to obtain unformatted input. (See also BINARY).

MULTIDATA: the qualifier required to load different data on each processing node. (See also MULTIDATA).

io_list: the list of the items to read. (See also I/O LISTS).

Description:

The READ statement transfers data from an external medium to the variables of an executing program.

If not specified, data are read from the user terminal. Apart from this case, a read statement must refer explicitly to a channel number, identifying the file from which data are to be transferred. When a read statement is performed on a floating point variable the same value is loaded on each node of the machine.

Floating point data can be loaded only in a particular node or in a slice of nodes adding a node specification or a slice specification to the read statement.

The multidata qualifier is used to load different data on each processing node.

Unformatted data input may be obtained by using the binary qualifier.

The multidata and binary qualifiers can be added in any order (i.e. both syntaxes multidata binary and binary multidata are allowed).

See also OPEN FOR READING, I/O LISTS.

Examples:

integer i

real a, b, c, w, k, l, vect[20], real_vect[60]

read  a, b, c, i

!! a, b and c are loaded in all the nodes; i is loaded in

!! the ZCPU memory.

read [0,0,0] w, k, l

!! w, k and l are loaded only in the [0,0,0] node.

read [0,0,0] [1,1,1]   vect[10]

!! one copy of the 11th element of the vector is loaded

!! in each node in the first board.

read  from  6   [0,0,0] [1,1,1] multidata real_vect[0] : 60

!! a copy of the whole vector is loaded in each node in

!! the specified slice (first board).

read  from  7    binary   vect_3d[3, 3, 3]

!! the [3,3,3] element is read from a binary file.

## REALINDVOCE "REAL"§INDVOCE "REAL"§

Syntax:

REAL    real_var_name {, real_var_name}

Parameters:

real_var_name: the identifier of the variable to be declared as real.

Description:

This statement is used to declare one or more variables whose values are real.

When floating point data are declared, the storage space for a copy of them is reserved in the data memory of each node of the machine.

Examples:

real    a, vect[100], vect_2d[10,10]

!!  declaration of a real variable named a, of an array of

!!  100 real numbers named vect and of a bidimensional

!!  matrix of 10x10 real numbers called vect_2d.

## REAR

Description:

See RIGHT.

## RECORDINDVOCE "RECORD"§

Syntax:

RECORD    record_name

data_type  data_name {, data_name}

{data_type  data_name {, data_name}}

ENDRECORD

Declaration of record type variables:

record_name    name {, name}

Parameters:

record_name: the record type identifier.

data_type: the elementary data-types forming the record type. Real, complex, matrix and other record types are allowed.

data_name: the names of the fields composing the record.

name: the names of the variables of the new type that are to be declared. Each of them is composed of as many fields as declared in the record statement.

Description:

The RECORD statement is used to declare a new type starting from elementary data types.

It can be used to build compound data structures, formed by more than one variable (also of different data-types).

Only record types composed of floating point fields are allowed; it is therefore possible to define a record type starting from real and complex variables.

A record type may also be composed of other record and matrix types.

Sums and differences between omologue records are introduced by default when you define a record type, as sums and differences between corresponding fields.

However, you can overload the default definition if you want to assign a different meaning to the "+" and "-" operators.

After declaring a new record type you can:

-  use it to build new aggregate data types;

-  define register and temporary variables of the new type;

-  define arrays of the new type;

- pass variables of the new type as arguments to a subroutine.

Record type variables cannot be passed to subroutines as values.

You can access the single fields forming a record variable by the use of the dot notation (".").

See also ENDRECORD, MATRIX.

Examples:

record    point

real    x, y, z

endrecord

!! definition of the point type, composed of 3 real fields.

point    p1, p2

!! p1 and p2 are point type variables; each of them is

!! composed of 3 real numbers.

p1.x = 3.4

!! The value of the first field of p1 is 3.4.

!! A record may be composed of variables of different

!! types:

record mixed

real r

complex z

endrecord

mixed m

!! m is a mixed type variable, composed of a real and

!! a complex number.

record    line

point    p1, p2

endrecord

!!  definition of a new data type named line, composed of

!!  two point type variables (six real fields).

line  L

!!  L is a line type variable.

!!  The dot notation enables you to address the single fields

!!  forming m and L:

m.z.re = 3.5

L.p1.x = 1.3

L.p1.y = 2.8

L.p1.z = 6.7

## REGISTERINDVOCE "REGISTER"§

Syntax:

REGISTER    data_type   var_name {, var_name}

Parameters:

data_type:  real, complex or previuosly defined record and matrix types.

var_name: the name of the floating point variables to be declared as register.

Description:

The REGISTER statement is used to declare a set of floating point variables whose values are stored only in the MAD processor registers, without being automatically saved in the data memory. In this way no unnecessary memory accesses are performed during execution. Initializing register variables and saving their values is up to the user.

Register variables preserve their content till the end of the "optimization block". In fact every source operation causing a branch or an entry point at the assembler level, produces the loss of the registers variables values. It is therefore necessary to initialize them again and to save their content when needed.

Record and matrix type variables can be declared as register.

Arrays cannot be declared as register. Integer and multindex register variables are not allowed.

See also EXTRACT, REPLACE and OPTIMIZATION BLOCKS.

Examples:

register   real    a, energy

register   complex   b, wave_function, psi

record    point

real    x, y, z

endrecord

register  point    a

!!  a is a register variable of the predefined point type

matrix      real     R_M.[10]

register    R_M    M1, M2

!! M1 and M2 are register variable of the predefined R_M

!! type.

## REPEATINDVOCE "REPEAT"§

Syntax:

REPEAT

block

AS LONG AS (global_condition)

Parameters:

global_condition: a condition defined on global integer variables.

A global condition can also be obtained using the ALL, NONE and ANY operators, which analyze the simultaneous status (on all the processing nodes) of a condition defined on local floating point variables.

Global conditions are defined by means of relational operators, comparing numerical values or expressions.

Conditions mixing integer and floating point data are not allowed.

block: a block of executable statements.

Description:

A block of instructions is executed iteratively, as long as the global condition is true.

The block is executed at least once. The expression of the global condition is evaluated at the end of each execution. If the condition is verified the statements forming the block are executed again; if it is false, control transfers to the instruction immediately following the as long as statement.

See also AS LONG AS, WHILE, ANY, ALL, NONE, RELATIONAL OPERATORS and LOCAL TO GLOBAL FLOW.

Examples:

repeat

write i

i = i + 1

f = (g - h)/e

as long as ( i<j)

## REPLACE INDVOCE "REPLACE "§

Syntax:

REPLACE   M-name   INTO   array_name[n]

Parameters:

M_name: the identifier of a matrix-type variable that has been declared as register. It is composed of N items.

array_name[n]: an array element.

Description:

The REPLACE statement copies a block of data from a matrix type register variable into an array. It is used to download groups of data stored in the MAD registers into contiguous memory locations. The array item specified in the statement selects the location in the array where the data block starts.

See also EXTRACT, MATRIX and REGISTER.

Examples:

matrix     complex    C.[5]

register    C     C2

complex      v_c[10]

replace    C2    into   v_c[1]

!!  the 5 complex numbers forming C2 are copied

!!  into v_c. The first number is loaded in v_c[1]

matrix      real    M_2d.[6,6]

register     M_2d     M2

real      v_2d[10,10]

replace   M2    into   v_2d[0, 0]

!! the elements of M2 are copied into v_2d.

## RIGHTINDVOCE "RIGHT"§, LEFTINDVOCE "LEFT"§, REARINDVOCE "REAR"§, FRONTINDVOCE "FRONT"§, UPINDVOCE "UP"§, DOWNINDVOCE "DOWN"§

Syntax:

array_name[n1, n2,...] = array_name[n1, n2, ..

+ keyword]

Parameters:

array_name: an array identifier.

n1, n2,...: integer numbers, identifying an array element. They must have values within the bounds of the associated dimension. The 0 value of each subscript identifies the first element of the array, while the N-1 value refers to the last one.

keyword: RIGHTINDVOCE "RIGHT"§, LEFTINDVOCE "LEFT"§, REARINDVOCE "REAR"§, FRONTINDVOCE "FRONT"§, UPINDVOCE "UP"§ or DOWNINDVOCE "DOWN"§.

Description:

Every processing node can access data stored on the nodes at its right, left, up, down, front and rear.

The RIGHT, LEFT, UP, DOWN, FRONT and REAR keywords can be added to the address of an array in order to access the data stored in  one of the neighbouring nodes.

The generated address is the same for all the nodes in the machine. Therefore if a processor performs an access to the processing node at its right (left, up, down, front or rear), all the other processor do the same at the same time.

Examples:

real v[4], m[3,3]

complex c_v[5]

v[0] = v[1 + right]

v[2] = v[3 + left]

m[0,0] = m[0,0 + up] + m[0,0 + down]

c_v[2].re = c_v[1 + rear].im + c_v[0 + front].re

## RRETURN

Syntax:

RRETURN  var_name

Parameters:

var_name: a variable identifier.

Description:

RRETURN is used within the definition of Zz statements and operators. An internal variable is created at the beginning of Zz action and a value is assigned to it every time the RRETURN statement is used. At the end the final value of this variable is returned.

If RRETURN is used more than once inside a single statement, only the final value of the variable is returned at the end. The Zz action is always executed till the last curly bracket. A RRETURN statement does not end the parsing of the action.

See also OPERATORS, _EXPR, _TERM, _FACT, _FUN and RRETURN in Part I.

Examples:

/real_fun-> " | " real_expr^x " | " {

temporary real _ris

_ris = x

where (x < 0.0)

_ris = - x

endwhere

rreturn _ris

}

!! returns the absolute value of x.

!! WRONG EXAMPLE:

/real_fun-> " | " real_expr^x " | " {

temporary real _ris

_ris = x

where (x >= 0.0)

rreturn _ris

elsewhere

rreturn -_ris

endwhere

}

!! not correct because rreturn has been used twice:

!! only the final value of _ris is returned.

## SIGNINDVOCE "SIN"§

Syntax:

/include <math>

SIGN(arg)

Parameters:

arg: an integer or real expression.

Description:

The SIGN function returns the sign INDVOCE "sine "§of the argument.

It returns:

1  if the argument is positive;

-1 if the argument is negative;

0  if the argument value is 0.

The result is always of the same data-type as the argument.

Examples:

integer i, j, k, l

i = 0

j = 6

k = -9

l = sign(i)			!! l = 0

l = sign(j)			!! l = 1

l = sign(k)		!! l = -1

real a, b, c, d

a = 0.0

b = 3.0

c = -7.0

d = sign(a)		!! d = 0.0

d = sign(b)		!! d = 1.0

d = sign(c)		!! d = -1.0

## SININDVOCE "SIN"§

Syntax:

/include <math>

SIN(arg)

Parameters:

arg: a real expression.

Description:

The SIN function returns the sine INDVOCE "sine "§of the argument.

The argument is expressed in radians.

Examples:

real a, b

b = sin(a)

## SIN_1OCT

Syntax:

/include <math>

SIN_1OCT(arg)

Parameters:

arg: a real expression.

Description:

The SIN_1OCTINDVOCE "SIN_1OCT"§ function returns the sine of the argument in the [0, p/4] range.

The argument is expressed in radians.

Examples:

real a, b

b = sin_1oct(a)

## SINHINDVOCE "SIN"§

Syntax:

/include <math>

SINH(arg)

Parameters:

arg: a real expression.

Description:

The SINH function returns the hyperbolic sine INDVOCE "sine "§of the argument.

Examples:

real a, b

b = sinh(a)

## SIZEOFINDVOCE "SIZEOF"§

Syntax :

SIZEOF(data_type)

Parameters:

data_type: integer, real, complex, or a record or matrix type defined by the user.

Description:

The SIZEOF operator returns an integer number, whose value  equals the number of integer or floating point words which compose the data-type passed as argument. It is evaluated at compilation time.

It may be used in the declaration of integer constants or as the start or end value for a control variable in a  /for  loop.

Examples:

constant c_i = sizeof(integer)                             !! c_i = 1

constant c_r = sizeof(real)                                  !! c_r = 1

constant c_c = sizeof(complex)                          !! c_c = 2

record point

real x, y, z

endrecord

constant n = sizeof(point)                                       !! n = 3

matrix complex C8_M.[8]

C8_M   C1, C2, C3

!! sizeof(C8_M) = 16

/for i = 0 to sizeof(C8_M) - 1 {

C3.[i] = C1.[i] + C2.[i]

}

## SQRTINDVOCE "SQRT"§

Syntax:

/include <math>

SQRT (arg)

Parameters:

arg: a real expression.

Description:

The SQRT function returns the square root INDVOCE "square root "§of the argument.

Examples:

real a, b

b = sqrt(a)

!! b = µ §

## STACKINDVOCE "STACK"§

Syntax:

STACK   data_type      var_name {,var_name}

Parameters:

data_type: integer, real, complex.

var_name: a variable identifier.

Description:

Static local variables are not reentrant, that is their values are not retained when a subroutine calls itself. The STACK prefix is used to declare variables that preserve their content in recursive subroutines.

The variables declared in this way are allocated on the stack. At the end of each execution of a recursive subroutine, the stack variables assume again the values obtained in the preceding one.

Examples:

stack integer n1, n2

stack real a, b, c

## SUBROUTINE

Syntax:

SUBROUTINE    sub_name  ({, param_decl})

block

END

Parameters:

sub_name: the subroutine identifier.

param_decl: declaration of a formal parameter, composed of the parameter type and name.

Subroutine parameters can be integer, real and complex variables or arrays; also matrix and record type variables are allowed.

block: a block of executable statements.

Description:

A subroutine is a subprogram, identified with a name. The SUBROUTINE statement begins the subprogram, declaring the formal parameters.

The CALL statement is used to invoke a subroutine.

Variables declared within the subroutine instruction block are allocated by default in the static data memory and become undefined when the control is returned to the calling program.

The variable names are local to the subroutine; it is then possible to use the same names for local and global variables; local variables hyde the global ones which have the same names.

It is not possible to use the same name for local and global variables of different types and for variables of the same type which are stored in a different way (i.e. a static variable and a register one).

The static local variables are not reentrant, that is they don't preserve their values when a subroutine calls itself; in order to obtain reentrant variables the STACK prefix has to be used in the variable declaration statement.

In order to perform the same block of instructions on different type of data (i.e. integer, real, complex or record and matrix types previously defined by the user), the TAO language allows to write more than one subroutine having the same name but arguments of different type. The compiler recognises which subroutine to call depending on the actual argument types.

See also CALL, STACK and END.

Examples:

real a, b, c

real p1x, p1y, p2x, p2y , dist, dist1

real V[5], W[5], Q[5]

complex z1, z2

call product(V, W, Q)

call mod(p1x, p1y, p2x, p2y, dist)

call mod(z1, z2,  dist1)

!! Declaration of a local variable having the same name

!! as a previously declared global one:

subroutine product(real V[5], real W[5], real Q[5])

real a

a = 35.2

do i = 0, 4

Q[i] = V[i]*W[i]

Q[i] = Q[i]/a

enddo

end

!!  two subroutines having the same name but

!!  arguments of different type:

subroutine mod (real x1, real y1, real x2, real y2, real ris)

ris = (x2 - x1)*(x2 - x1) + (y2 - y1) * (y2 - y1)

ris = sqrt(ris)

end

subroutine mod (complex z1, complex z2, real ris)

ris = (z2.re - z1.re)*(z2.re - z1.re) +...

(z2.im - z1.im)*(z2.im - z1.im)

ris = sqrt(ris)

end

## TANINDVOCE "SIN"§

Syntax:

/include <math>

TAN(arg)

Parameters:

arg: a real expression.

Description:

The TAN function returns the tangent INDVOCE "sine "§of the argument.

The argument is expressed in radians.

Examples:

real a, b

b = tan(a)

## TANHINDVOCE "SIN"§

Syntax:

/include <math>

TANH(arg)

Parameters:

arg: a real expression.

Description:

The TANH function returns the hyperbolic tangent INDVOCE "sine "§of the argument.

Examples:

real a, b

b = tanh(a)

## TEMPORARYINDVOCE "TEMPORARY"§

Syntax:

TEMPORARY  data_type  var_name  {, var_name}

Parameters:

var_name: the identifier of the variable to be declared as temporary.

data_type:  integer, real, complex or previuosly defined record and matrix types.

Description:

The TEMPORARY statement is used to declare variables that are local to a single statement.

The values of temporary integer variables are stored in the ZCPU data memory.

The values of floating point temporary variables are stored only in the MAD processor registers, without being automatically saved in the data memory. In this way unnecessary memory accesses are not performed during the execution of the statement.

Temporary variables become undefined each time control is returned to the calling program unit. They have to be initialized within the statement in which they are used; in this way their values are available each time the statement is called. Saving their content if necessary is up to the user.

It is possible to declare temporary variables of previously defined record  and matrix types.

See also Zz STATEMENTS (Part I) and REGISTER.

Examples:

!! Definition of the absolute value of a complex number:

/real_fun -> " | " complex_expr^_a " | " {

temporary  real  _ris

_ris = _a.re*_a.re + _a.im*_a.im

_ris = sqrt(_ris)

rreturn _ris

}

## TRUNCINDVOCE "FINT"§

Syntax:

/include <math>

TRUNC(arg)

Parameters:

arg: a real expression.

Description:

The TRUNC function returns a real number having the same sign as the argument; its value equals the largest integer whose absolute value does not exceed that of the argument.

Examples:

real a, b, c, d, e, f, g, h

a = 3.2

b = 3.9

e = -3.3

g = -3.8

c = trunc(a)	!! c = 3.0

d = trunc(b)	!! d = 3.0

f = trunc(e)	!! f = -3.0

h = trunc(g)	!! h = -3.0

## TYPEINDVOCE "FINT"§

Description:

See IS MADE OF.

## UP

Description:

See RIGHT.

## WHEREINDVOCE "WHERE"§

Syntax:

1)

WHERE (local_condition)

block

ENDWHERE

2)

WHERE (local_condition)

block 1

ELSEWHERE

block  2

ENDWHERE

Parameters:

local_conditionINDVOCE "local condition"§: a condition defined on local floating point variables. Local conditions are defined using relational operators, comparing floating point numerical values or expressions. Logical operators can be used to combine conditions together.

block, block1, block2: blocks of executable statements.

Description:

The WHERE statement enables you to execute an instruction or a block of instructions only on those nodes in which the local conditions is verified.

This statement acts disabling the assignments on those nodes where the condition is false.

In this way the synchronization among nodes is preserved.

The local conditions control the execution of the block of statements locally on each node, without changing the global program flow.

The TAO language provides the two shown above forms of where statements:

1) The instructions contained in the block are executed on those nodes where the local condition is true.

2) The instructions of block 1 are executed on those nodes where the local condition is true, the instructions of block 2 are performed on those nodes where the condition is false.

See also ELSEWHERE, ENDWHERE, RELATIONAL OPERATORS and LOGICAL OPERATORS.

Examples:

real q, r, w, f

where (q > r  | |   q > w )

f = 6.0e-6

endwhere

real a, b, c, d

where  (a > b &&  a ! = c)

d = (a - b)/(a - c)

elsewhere

d = (b - a)/(c - a)

endwhere

## WHILEINDVOCE "WHILE"§

Syntax:

WHILE (global_condition)

block

ENDWHILE

Parameters:

global_condition: a condition defined on global integer variables.

A global condition can also be obtained using the ALL, NONE and ANY operators, which analyze the simultaneous status (on all the processing nodes) of a condition defined on local floating point variables.

Global conditions are defined by means of relational operators, comparing numerical values or expressions.

Conditions mixing integer and floating point data are not allowed.

block: a block of executable statements.

Description:

A block of instructions is executed iteratively while the global condition is true. The expression of the global condition is evaluated at the beginning of each execution. If the condition is true, the statements that form the block are executed; if it is false, control transfers to the instruction immediately following the endwhile statement.

See also ENDWHILE, REPEAT, ANY, ALL, NONE, RELATIONAL OPERATORS and LOCAL TO GLOBAL FLOW.

Examples:

while (i > k)

i = i + 1

write j + i

a = b*c - d*f

endwhile

while (i < j)

write i

i = i + 1

f = (g - h)/e

endwhile

## WRITE

INDVOCE "WRITE"§Syntax:

Writing to the terminalINDVOCE "Writing to the terminal"§:

WRITE [nod_spec] [nod_spec] [BINARY] io_list

Writing on a file:

WRITE ONINDVOCE "WRITE ON"§  file_unit  [nod_spec] [nod_spec] ...

[BINARY]   io_list

Parameters:

file_unit: an integer constant in the [2;15] range. It specifies the channel number assigned to the file. (See also  OPEN FOR WRITING).

node_spec: three integer  numbers in square brackets, separated by commas, identifying a processing node.

(See also NODE SPECIFICATION and SLICE SPECIFICATION).

BINARY: the qualifier required to obtain unformatted output. (See also BINARY).

io_list: the list of the items to write. (See I/O LISTS).

Description:

The WRITE statement is used to write data either to a file or to the user terminal.

If not specified, data are written to the user terminal. Apart from this case, a write statement must refer explicitly to a channel number, identifying the file to which data are to be transferred.

It is possible to output data from a particular node or from a slice of nodes, adding a node specification or a slice specification to the write statement. If node and slice specifications are omitted floating point data are output from the default node in the default processing board.

Unformatted data output may be obtained by using the binary qualifier.

See also OPEN FOR WRITING, I/O lLISTS.

Examples:

integer i

real a, b, c, w, k, l, vect[20], real_vect[60]

write  i, a, b, c

!! the values of i, a, b and c are written to the user terminal.

write [0,0,0] w, k, l

!! the values of w, k and l stored in the [0,0,0] node are

!! written to the user terminal.

write [0,0,0] [1,1,1]   vect[10]

!! the values of the 11th element of the vector stored in

!! all the nodes in the first board are written to the terminal.

write  on  6   [0,0,0] [1,1,1]   real_vect[0] : 60

!! the values of all the 60 vector items stored in all the nodes

!! in the specified slice (first board) are written on a file.

write  on   7  binary   vect_3d[3, 3, 3]

!! the [3,3,3] element is written in the machine binary

!! format on an external file.

write "Hello world"

!! Hello world is written on the terminal.

## !: LOGICAL  NOT INDVOCE "NOT "§OPERATOR  INDVOCE "!"§

Syntax:

!  logical_condition

Parameters:

logical_conditionsINDVOCE "logical condition"§: a local or global condition.

Description:

The " ! " operator executes a logical NOT on the specified logical condition.

Examples:

if (! (i = = j && j = = 0))

a[i] = a[j]*(1/j) + 1/(i - j)

else

a[i] = 0

endif

## &INDVOCE "&"§ INDVOCE "THE & SIGN"§

Syntax:

WRITE [nod_spec] [node_spec] [BINARY] io_list &

WRITE [nod_spec] [nod_spec] [BINARY] io_list

Description:

The & sign at the end of a write statement (after the i/o list), allows the next write statement to continue writing on the same line. Otherwise a line feed would be inserted by default at the end of  each write statement.

Using the & sign it is possible to write data stored in different processing nodes or slices on the same line.

Examples:

write " I don' t want to start" &

write "a new line"

!!  The result is:

!!  I don' t want to start a new line

!!  The result of the operation without the & sign would be:

!!  I don' t want to start

!!  a new line

do i = 0, 1

do j = 0, 1

do k = 0, 1

write [i, j, k] a &

enddo

enddo

enddo

!! the a values stored on the different processing nodes are

!! written on the same line

## &&: LOGICAL  ANDINDVOCE "AND"§ OPERATOR

Syntax:

logical_condition    &&    logical_condition

Parameters:

logical_conditionsINDVOCE "logical condition"§: local or global conditions. The conditions must either be both global (integer) or both local (floating point).

Description:

The "&&" operator executes a logical AND between the results of the two specified logical conditions.

Examples:

where ( arg > = 0.0 && b ! =3.0)

a = sqrt(arg)/(b - 3.0)

elsewhere

a = 0.0

endwhere

## .REINDVOCE ".RE"§ and .IM: REAL AND IMAGINARY PART   EXTRACTION POSTFIX OPERATORS  INDVOCE ".IM"§INDVOCE "REAL AND IMAGINARY PART EXTRACTION POSTFIX OPERATORS\\:  .RE and ."§

Syntax:

compl_var.RE

compl_var.IM

Parameters:

compl_var: the identifier of a previously declared complex variable.

Description:

The .RE and .IM postfix operators are used to select the real or the imaginary part of a complex variable.

Examples:

complex    z1, c[4]

z1 = (1.0 , 2.0)

c[0] = (0.0, 1.0)

z1.re = 1.0

z1.im = 2.0

c[0].re = 0.0

c[0].im = 1.0

## /BEEP

INDVOCE "/BEEP"§Syntax:

/BEEP ["message"]

Parameters:

message: a message to be written on the screen at compilation time. It must be quoted.

Description:

This statement is evaluated at compilation time.

It prints on the screen:

-  a sequential number or optionally the message;

-  the SUN CPU current time;

-  the name of the input file;

-  the current source code line.

Examples:

/BEEP "The Beginning!"

/BEEP "The End!"

## /FORINDVOCE "/FOR"§

Syntax:

/FOR index_var = start_val TO stop_val [STEP step_val]{

block

}

Parameters:

index_var: the integer Zz variable which controls the execution of the /for loop. It is not necessary to declare it. It is possible to refer to the control variable inside the loop.

start_val: the initial value of the control variable. It is an integer value or an expression generating integer numbers.

stop_val: the final value of the control variable. It is an integer value or an expression generating integer numbers.

step_val: the value by which the control variable is incremented after each execution of the /for loop. If omitted it is set to 1 by default. It is an integer value or an expression generating integer numbers.

block: a block of executable statements.

Description:

A /FOR statement executes a block of instructions iteratively until the value of an integer control variable equals a terminal value. It is not necessary to declare the control variable before using it.

The control variable is a Zz variable; it is alive only at compilation time and is not saved on the ZCPU data memory. It is not possible to assign the value of an integer TAO variable to the control variable.

The statements contained in a /for loop are evaluated at compilation time.

The compiler expands the instructions contained in the loop without producing jumps at the assembler level; therefore, the section does not terminate at the end of each execution and the compiler succeeds in filling the MAD pipeline. Moreover, register and temporary variables do not lose their values.

For these reasons, improvements in the optimization of programs may be often achieved by replacing DO-loops by /FOR loops.

See also OPTIMIZATION BLOCKS and Zz statements in Part I and DO in Part II.

Refer to "Zz the TAO engine" manual.

Examples:

real a[10], b[10], c[10]

/for i = 0 to 9 {

a[i] = b[i] * c[i ]

}

!!     this is equivalent to the following block of code:

!!

!!     a[0] = b[0] * c[0]

!!     a[1] = b[1] * c[1]

!!     ...

!!     a[9] = b[9] * c[9]

!!    The machine succeeds in filling the MAD pipeline.

!!     A do loop could be used to performe the same

!!     calculations:

do i = 0, 9

a[i] = b[i] * c[i]

enddo

!! But in this case the section starts and ends at the same

!! instruction and the machine is not able to fill the mad

!! pipeline; the jump due to the DO loop clears out the

!! register file each time the ENDDO statement is reached.

matrix  real  R_M.[10]

R_M     M, N, Q

/for j = 0 to sizeof(R_M) - 1 {

M.[j] = N.[j] + Q.[j]

}

real a, b, c

real v[100], P[100], R[100]

/for k = 0 to 99 step 10 {

v[k] = P[k]/(a + b) + R[k]*c

}

integer m

/for i = 1 to 100 {

i = m				!! FORBIDDEN

}

## /FOREACH

INDVOCE "/FOREACH"§Syntax:

/FOREACH  variable IN list {

action

}

Parameters:

variable: the identifier of a Zz variable which takes the value of each item in the list.

list: the Zz list identifier.

action: one or more executable statements.

Description:

The /FOREACH statement executes the specified action once for each item in the list. The variable specified in the statement takes the value of each of the items.

See also Zz STATEMENTS and LISTS in Part I.

Refer to "Zz, the TAO Engine" manual.

Examples:

list = {1 2 3 4 5}

/foreach item in list {

write "the number is", item

}

## /IFINDVOCE "/INCLUDE"§

Syntax:

/IF    logical_condition {

block

}

Parameters:

logical_condition: a condition defined on a Zz variable.

block:  a block of executable statements.

Description:

The /IF statement allows to control the parsing during compilation. The compiler expands the instructions written between curly brackets if the condition is true.INDVOCE "language extension libraries" \r "74/75"§

See also Zz STATEMENTS in Part I.

Refer to "Zz, the TAO Engine" manual.

Examples:

INDVOCE "Language Extension Libraries"§/diff = size_a - size_b

/if diff > 0 {

/for j = 0 to diff - 1{

v.[j] = w.[j] + q.[j]

}

}

!! The compiler expands the instruction contained in the

!! /for loop if diff value is > 0.

## /INCLUDEINDVOCE "/INCLUDE"§

Syntax:

/INCLUDE    library_name.ext

/INCLUDE    "/path/library_name.ext"

Assuming that the extension is .hzt and that the file is in the current directory:

/INCLUDE    library_name

Referring to a file contained in the directory $TAOLIB, defined in the UNIX environment:

/INCLUDE  <library_name>

It is also possible to use environment variables in the include path. See later on in the examples.

Parameters:

library_name:  the name of a language extension library.

.hzt: the standard extension for a TAO library file.

.ext: other extensions.

path: a path different from the current directory.

Description:

The /INCLUDE statement allows to use a particular language extension library.

The TAO environment is composed of a number of predefined language structures and statements contained in some language extension librariesINDVOCE "language extension libraries" \r "74/75"§. Some of these libraries are automatically included when the compiler is invoked (i.e. $TAOLIB/std.hzt, $TAOLIB/stdio.hzt), while the others must be included by the user.

It is possible to write down and include new language extension libraries, containing new structures, statements and syntax extensions defined by the user.

See also  <MATH>, <RANDOM>, <STDIO>.

Examples:

/include <math>INDVOCE "Language Extension Libraries"§

/include <random>

/include <qcd>

/include  my_library.hzt

/include  " lib_dir/new_library.hzt"

!! The Zz parser allows to access the environment variables !! using the notation $(<env_var>), e.g. $(TAOLIB).

!! It can be useful to insert the following Zz rules at the

!! beginning of the program:

/filename-> string_t^_env "/" filename^_name {

/return  _env&"/"&_name

}

!! so that the following statements can be accepted:

/include $(MYDIR)/my_library.hzt

/include $(MYDIR)/subdir/my_library.hzt

/include /home/phys/my_lib

## /PRINTINDVOCE "/PRINT"§

Syntax:

/PRINT  arg_list

Parameters:

arg_list: the list of the items to be printed on the screen. They have to be separated by commas.

Description:

The /print statement is used to print something on the screen at compilation time. Arguments of any basic type may be printed, together with quoted strings, integer  and floating point numbers and expressions.

TAO variables, of course, cannot be printed.

See also Zz STATEMENTS.

Refer to "Zz, the TAO Engine" manual.

Examples:

/print "first part of compilation O.K.!"

## _EXPR

Description:

See OPERATORS and _EXPR,_TERM, _FACT and _FUN in PART I .

## _FACT

Description:

See OPERATORS and _EXPR,_TERM, _FACT and _FUN in PART I .

## _FUN

Description:

See OPERATORS and _EXPR,_TERM, _FACT and _FUN in PART I .

## _TERM

Description:

See OPERATORS and _EXPR,_TERM, _FACT and _FUN in PART I .

## | | : LOGICAL  ORINDVOCE "OR"§ OPERATOR   INDVOCE "| |"§INDVOCE "LOGICAL  OR OPERATOR\\:   | |"§

Syntax:

logical_condition    | |    logical_condition

Parameters:

logical_conditionINDVOCE "logical condition"§: local or global conditions. The conditions must either be both global (integer) or both local (floating point).

Description:

The " | | " operator executes a logical OR between the results of the two specified conditions.

Examples:

if ( i > 3 | |  j < 5)

v[i, j] = b + c

endif

## ~: COMPLEX CONJUGATION OPERATORINDVOCE "COMPLEX CONJUGATION OPERATOR"§  INDVOCE "~"§

Syntax:

compl_var~

Parameters:

compl_var: the identifier of a predeclared complex variable.

Description:

The complex conjugation postfix operator "~" produces the complex conjugate of a complex number.

Examples:

complex    z1, z2

z1 = (1.0, 1.0)

z2 = z1~

!! z2 is the complex conjugate of z1.

!! z2 = (1.0, -1.0)